Electrical and Computer Systems Engineering Department

Communications Lab

ENEE 4103

QPSK Report

Students Names: Aseel Awad                 1130776

Sharehan Badwan        1130167

Sarah Usta                     1130501

Instructor:Dr. Wael Hashlamoun

Section.No. 3

Assistant:Eng. Ahmad Faza'

# Abstract

The aim of the project is to build and simulate a modulation technique which is QPSK (Quadratic Phase Shift Keying) modulation that is used to transmit binary data of ones and zeros. Moreover, noticing the effect of adding AWGN to the modulated signal and what happens to the  bit error rate and symbol error rate at different values of noise.

Table of Contents

Abstract	II

1. Theory:	1

1.1 Quadratic phase shift keying	1

1.2 QPSK Modulation: [3]	1

1.3 QPSK  Demodulation	2

1.4 Additive white Gaussian noise [4]	3

1.5 Symbol rate [5]	3

1.6 Bit rate [5]	3

1.7 Symbol rate vs Bit rate[6]	4

2. System and Components	5

2.1 System Design	5

2.2 Components	5

1.   The Bernoulli Binary Generator block	5

2. QPSK Modulator Baseband Block	6

3. The AWGN Channel block	8

4. The QPSK Demodulator Baseband Block	9

5. Error Rate Calculation Block	10

6. The Display block	11

7. Unbuffer Block	11

8.  Scope Block	11

9. Time Scope Block	11

3. Simulation Results	13

4. Calculation Results	15

5. Conclusion	16

6. References	17

# 1. Theory:

Most data communications over networks occurs via serial-data transmission. Data bits transmit one at a time over some communications channel, such as a cable or a wireless path. [1]

## 1.1 Quadratic phase shift keying

QPSK (Quadrature Phase-Shift Keying) is a digital modulation format that modulates data onto the carrier by changing the phase of the carrier. The data is demodulated by comparing the received signal to the carrier and sampling the phase at a constant rate (symbol rate). [2]

For QPSK, there are four possible phase values (± pi/4 and ± 3pi/4) and therefore log2(4) = 2 bits of data can be transmitted on each symbol.A sample QPSK constellation diagram is shown below

Figure 1.1: QPSK constellation

## 1.2 QPSK Modulation: [3]

In digital modulation techniques a set of basis functions are chosen for a particular modulation scheme.Generally the basis functions are orthogonal to each other. Basis functions can be derived using ‘Gram Schmidt orthogonalization’ procedure.Once the basis function are chosen, any vector in the signal space can be represented as a linear combination of the basis functions. In Quadrature Phase Shift Keying (QPSK) two sinusoids (sin and cos) are taken as basis functions for modulation. Modulation is achieved by varying the phase of the basis functions depending on the message symbols. In QPSK, modulation is symbol based,where one symbol contains 2 bits. The following equation outlines QPSK modulation technique. si(t)=2EsT−−−−√cos(2πfct+(2n−1)π4),n=1,2,3,4 When n=1, the phase shift is 45 degrees. This is called pi/4 QPSK.The constellation diagram of QPSK will show the constellation points lying on both x and y axes.This means that the QPSK modulated signal will have an in-phase component (I) and also a quadrature component (Q). This is because it has only two basis functions. A QPSK modulator can be implemented as follows. A demultiplexer (or serial to parallel converter) is used to separate odd and even bits from the generated information bits. Each of the odd bits (quadrature arm) and even bits (in-phase arm) are converted to NRZ format in a parallel manner. The signal on the in-phase arm is multiplied by cosine component and the signal on the quadrature arm is multiplied by sine component. QPSK modulated signal is obtained by adding the signal from both in-phase and quadrature arm.

Figure 1.2: QPSK modulator

1.3 QPSK  Demodulation [3]

For QPSK demodulator , a coherent demodulator is taken as an example. In coherent detection technique the knowledge of the carrier frequency and phase must be known to the receiver. This can be achieved by using a PLL (phase lock loop) at the receiver. A PLL essentially locks to the incoming carrier frequency and tracks the variations in frequency and phase. For the following simulation , a PLL is not used but instead we simple use the output of the PLL. For demonstration purposes we simply assume that the carrier phase recovery is done and simply use the generated reference frequencies at the receiver (cos(ωt)) and (sin(ωt)). In the demodulator the received signal is multiplied by a reference frequency generators (cos(ωt)) and (sin(ωt)) on separate arms (in-phase and quadrature arms). The multiplied output on each arm is integrated over one bit period using an integrator. A threshold detector makes a decision on each integrated bit based on a threshold. Finally the bits on the in-phase arm (even bits) and on the quadrature arm (odd bits) are remapped to form detected information stream. Detector for in-phase arm is shown below. For quadrature arm the below architecture remains same but sin(ωt) basis function must be used instead.

Figure 1.3: QPSK Demodulator

## 1.4 Additive white Gaussian noise [4]

Additive white Gaussian noise (AWGN) is a basic noise model used in Information theory to mimic the effect of many random processes that occur in nature. The modifiers denote specific characteristics:

Additive because it is added to any noise that might be intrinsic to the information system.

White refers to the idea that it has uniform power across the frequency band for the information system. It is an analogy to the color white which has uniform emissions at all frequencies in the visible spectrum.

Gaussian because it has a normal distribution in the time domain with an average time domain value of zero.

AWGN is often used as a channel model in which the only impairment to communication is a linear addition of wideband or white noise with a constant spectral density (expressed as watts per hertz of bandwidth) and a Gaussian distribution of amplitude. The model does not account for fading, frequency selectivity, interference, nonlinearity or dispersion. However, it produces simple and tractable mathematical models which are useful for gaining insight into the underlying behavior of a system before these other phenomena are considered.

## 1.5 Symbol rate [5]

In digital communications, symbol rate, also known as baud rate and modulation rate, is the number of symbol changes, waveform changes, or signaling events, across the transmission medium per time unit using a digitally modulated signal or a line code. The symbol rate is measured in baud (Bd) or symbols per second. In the case of a line code, the symbol rate is the pulse rate in pulses per second. Each symbol can represent or convey one or several bits of data. The symbol rate is related to the gross bitrate expressed in bits per second.

## 1.6 Bit rate [5]

In digital transmission, the number of bit errors is the number of received bits of a data stream over a communication channel that have been altered due to noise, interference, distortion or bit synchronization errors.

The bit error rate (BER) is the number of bit errors per unit time. The bit error ratio (also BER) is the number of bit errors divided by the total number of transferred bits during a studied time interval. BER is a unitless performance measure, often expressed as a percentage.

The bit error probability pe is the expectation value of the bit error ratio. The bit error ratio can be considered as an approximate estimate of the bit error probability. This estimate is accurate for a long time interval and a high number of bit errors.

## 1.7 Symbol rate vs Bit rate[6]

Symbols can be used to encode multiple bits.  Bits are a fundamental measurement of information.  Symbols are what is transmitted.

in  the early days ,  analog telephone modems weren't very complicated, so bps was equal to baud. That is, each symbol encoded one bit. Later, to make modems faster, communications engineers invented increasingly clever ways to send more bits per symbol.

# 2. System and Components

## 2.1 System Design

The whole system (Transmitter and Receiver) of QPSK is shown in figure below.

Figure 2.1: QPSK System

## 2.2 Components

The components (Blocks) used in the system are described and discussed bellow

### 1.   The Bernoulli Binary Generator block

The Bernoulli Binary Generator block generates random binary numbers using a Bernoulli distribution. The Bernoulli distribution with parameter p produces zero with probability p and one with probability 1-p. The Bernoulli distribution has mean value 1-p and variance p(1-p). The Probability of a zero parameter specifies p, and can be any real number between zero and one.

#### Parameters Settings in Bernoulli Binary Generator block

Probability of a zero

The probability with which a zero output occurs. Specify the probability as a scalar or row vector whose elements are real numbers between 0 and 1. The number of elements in the Probability of a zero parameter correspond to the number of independent channels output from the block. In this project, it was set to 0.5

Sample time

The time between each sample of a column of the output signal. I was set to 1 second

Samples per frame

The number of samples per frame in one channel of the output signal. Specify Samples per frame as a positive integer scalar. In the project it was set to 2. This means that in 1 second there is 2 samples of ones or zeros

Figure 2.1:Parameter Settings in Bernoulli Binary Generator

### 2. QPSK Modulator Baseband Block

The QPSK Modulator Baseband block modulates using the quaternary phase shift keying method. The output is a baseband representation of the modulated signal.

#### Integer-Valued Signals and Binary-Valued Signals

If  the Input type parameter  was set to Integer, then valid input values are 0, 1, 2, and 3. When Constellation ordering to Binary for input m the output symbol is

exp(jθ + jπm/2)

where θ represents the Phase offset parameter (see the following figure for Gray constellation ordering). In this case, the block accepts a scalar or column vector signal.

#### Parameters Settings

In this project, the Input type parameter is  Bit, then the input contains pairs of binary values. For this configuration, the block accepts column vectors with even lengths. The Phase offset parameter was set to π/4 then the block uses one of the signal constellations in the following figure, depending on whether the Constellation ordering parameter to Binary or Gray. However,  in this project  Constellation ordering  parameter is Gray.

Figure 2.2: Constellation ordering

Figure 2.1:Parameter Settings in QPSK Modulator Baseband

### 3. The AWGN Channel block

The AWGN Channel block adds white Gaussian noise to a real or complex input signal. When the input signal is real, this block adds real Gaussian noise and produces a real output signal. When the input signal is complex, this block adds complex Gaussian noise and produces a complex output signal. This block inherits its sample time from the input signal.

This block accepts a scalar-valued, vector, or matrix input signal with a data type of type single or double. The output signal inherits port data types from the signals that drive the block.

#### Parameters Settings

Mode

The mode by which the noise variance is specified: Signal to noise ratio (Eb/No), Signal to noise ratio (Es/No), Signal to noise ratio (SNR), Variance from mask, or Variance from port. In this project the Mode was set to Eb/No (dB) : The ratio of information (i.e., without channel coding) bit energy per symbol to noise power spectral density, in decibels. This field appears only if  Mode is set to Eb/No and it should be 2.

Number of bits per symbol

The number of bits in each input symbol. This field appears only if Mode is set to Eb/No.

Input signal power, referenced to 1 ohm (watts)

The mean square power of the input symbols (if Mode is Eb/No or Es/No) or input samples (if Mode is SNR), in watts. This field appears only if Mode is set to Eb/No, Es/No, or SNR.

Symbol period (s)

The duration of an information channel (i.e., without channel coding) symbol, in seconds. This field appears only if Mode is set to Eb/No or Es/No. It should be greater than the symbol period comes from the binary generator.

Figure 2.1:Parameter Settings in AWGN Block

### 4. The QPSK Demodulator Baseband Block

QPSK Demodulator Baseband Block  Demodulate the input signal using the quaternary phase shift keying method.

#### Parameters  Settings

The Output  type parameter was set  Bit, then the input contains pairs of binary values.. The Phase offset parameter was set to π/4. The Constellation ordering parameter was set to Gray

Figure 2.1:Parameter Settings in QPSK Demodulator Baseband Block

### 5. Error Rate Calculation Block

The Error Rate Calculation block compares input data from a transmitter with input data from a receiver. It calculates the error rate as a running statistic, by dividing the total number of unequal pairs of data elements by the total number of input data elements from one source.

Use this block to compute either symbol or bit error rate, because it does not consider the magnitude of the difference between input data elements. If the inputs are bits, then the block computes the bit error rate. If the inputs are symbols, then it computes the symbol error rate.

Output Data

This block produces a vector of length three, whose entries correspond to:

- The error rate

- The total number of errors, that is, the number of instances that an Rx element does not match the corresponding Tx element

- The total number of comparisons that the block made

### 6. The Display block

The Display block displays the following information:

• The bit error rate

• The number of errors

• The total number of bits that are transmitted

### 7. Unbuffer Block

Convert a frame to scalar samples output at a higher rate. It was used in this project to be able to calculate bit error rate and to show the input binary sequence.

### 8.  Scope Block

Display signals generated during simulation.

### 9. Time Scope Block

Display time-domain signals. Time Scope block accepts input signals with the following characteristics:

- Continuous or discrete sample time

- Real- or complex-valued

# 3. Simulation Results

Binary input Signal (Output of Bernulli Binary Generator) for 20s

Figure 3.1: Binary input Signal (Output of Bernulli Binary Generator) for 20s

Modulated QPSK  signal for 1000 sec

Figure 3.2: Modulated QPSK signal for 1000 sec

Modulated Signal with AWGN at -30db SNR

Figure 3.3: Modulated QPSK signal  with AWGN at -30db SNR

Demodulated QPSK  signal for 20sec and -30db SNR

Figure 3.4: Demodulated QPSK signal for 20sec and -30db SNRs

# 4. Calculation Results

The following table includes Bit Error Rate and Symbol Error Rate with Sequence of 1000 Symbol and Symbol Time 1 sec and different SNR values.

| SNR(db) | -30 | -20 | -10 | -8 | -6 | -4 | -2 | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BER | 0.503 | 0.4711 | 0.3503 | 0.3014 | 0.2475 | 0.1996 | 0.1337 | 0.08782 | 0.04591 | 0.01697 | 0.000998 | 0 | 0 |
| SER | 0.5025 | 0.4703 | 0.3498 | 0.3008 | 0.2469 | 0.1989 | 0.1339 | 0.08796 | 0.04598 | 0.01699 | 0.0009995 | 0 | 0 |

Figure 3.4: Bit Error Rate and Symbol Error Rate with Sequence of 1000 Symbol and Symbol Time 1

From the previous results, it can be concluded that both BER and SER decrease as SNR increase (it reaches 0 at SNR= 8). This is because the energy for the transmitted bits becomes larger than the energy for the noise signal. In addition, SER is greater than BER.

# 5. Conclusion

In this project QPSK modulation technique (Quadratic Phase Shift Keying) was built and simulated. This modulation is used to transmit sequence of binary data of ones and zeros. It deals Symbol rate (two bit) rather than Bit rate, then it modulates data onto the carrier by changing the phase of the carrier according to the transmitted symbol. The data is demodulated by comparing the received signal to the carrier and sampling the phase at a constant rate (symbol rate). The advantage of frame-based processing is that it can greatly increase the speed of a simulation. Moreover, the effect of adding AWGN to the modulated signal  was  studied by computing the bit error rate and symbol error rate at different values of noise.

# 6. References

- http://electronicdesign.com/communications/what-s-difference-between-bit-rate-and-baud-rate

- http://rfmw.em.keysight.com/wireless/helpfiles/89600B/WebHelp/Subsystems/digdemod/content/dlg_digdemod_fmt_qpsk.htm

- http://l.facebook.com/l.php?u=http%3A%2F%2Fwww.idc-online.com%2Ftechnical_references%2Fpdfs%2Felectronic_engineering%2FQPSK_modulation_and_Demodulation.pdf&h=7AQGqjQjT

- https://en.wikipedia.org/wiki/Additive_white_Gaussian_noise

- https://en.wikipedia.org/wiki/Symbol_rate

- https://en.wikipedia.org/wiki/Bit_error_rate