Prelab coummunication exp#1
Maher Saleem 1130258
section TR

- Write the mathematical expression for the demodulated signal.

Ans:

- Use MATLAB command and M files to draw the demodulated signal for the following three cases:
a. Ac=16v, modulation index=0.22, modulating signal frequency=800Hz

b. Ac=16v, modulation index=1, modulating signal frequency=800Hz.




c. Ac=16v, modulation index=1.85, modulating signal frequency=800Hz

In the second part , since the modulation index iss one , so the cosine is between -1 to 1 abs(1+ucos(wm)) is between 0 to 2  16* abs(1+ucos(wm)) between 0 to 32  we will receive the original message without distortion as shown in the figure above since we don’t cross the x axis

In the first part , since the modulation index is less than one , so the cosine is between -0.22 to 0.22  abs(1+ucos(wm)) is between 0.78 to 1.22  16* abs(1+ucos(wm)) between 12.48 to 19.52  we will receive the original message without distortion as shown in the figure above

b)
mue = 1;

Ac = 16;

fm = 800;

t = 1:0.00001:2;

output = Ac*abs(1+mue*cos(2*pi*fm*t));

plot(t , output , 'lineWidth',4)

grid

axis([1 , 1.01,-1,35])

a)
mue = 0.22;

Ac = 16;

fm = 800;

t = 1:0.00001:2;

output = Ac*abs(1+mue*cos(2*pi*fm*t));

plot(t , output , 'lineWidth',4)

grid

axis([1 , 1.01,10,20])

Discuss your result in each part .you must write the commands which are used in the Pre-lab

In the third  part , since the modulation index is larger than 1 , there will be a distortion  , so the cosine is between -1.85 to 1.85  abs(1+ucos(wm)) is between |-.85| to 2.85  16* abs(1+ucos(wm)) between 16*|-.85|  to 45.6  we will receive a distorted signal , since the absolute value will change the original sine shape

c)
mue = 1.85;

Ac = 16;

fm = 800;

t = 1:0.00001:2;

output = Ac*abs(1+mue*cos(2*pi*fm*t));

plot(t , output , 'lineWidth',2)

grid

axis([1 , 1.01,-1,50])