# Abstract

In this report, I will talk about how to calculate signal to noise ratio (SNR) for bath cases pre-detection and post-detection in these two modulation techniques DSB-SC and SSB, for the Adaptive Gaussian white Noise (AGWN).

Contents

I.	Abstract	1

III.	Introduction.	3

1.	Noise is communication systems.	3

2.	Signal to noise ratio.	3

3.	Additive white Gaussian noise.	4

4.	Noise is DSB-SC	4

5.	Noise in SSB	5

IV.	Procedure and discussion	5

1.	Noise in DSB-SC Modulation	5

2.	Noise in SSB Modulation	7

3.	SSB and DCB-SC with  -12db noise	8

V.	Conclusion	9

VI.	References	9

# Introduction.

## Noise is communication systems.

Noise is a random fluctuation in an electrical signal, a characteristic of all electronic circuits. Noise generated by electronic devices varies greatly, as it can be produced by several different effects. Thermal noise is unavoidable at non-zero temperature, while other types depend mostly on device types, which needs steep potential barrier or manufacturing quality and semiconductor defects, such as conductance fluctuations, including 1/f noise.

In communication systems, noise is an error or undesired random disturbance of a useful information signal in a communication channel. The noise is a summation of unwanted or disturbing energy from natural and sometimes man made sources. In general, noise is unwanted, but it can serve a useful purpose in some applications, such as random number generation or dither.

As the AM is LTI systems, we could use superposition principle. That implies

r(t) = s(t) + w(t). ….  (1)

Where s(t)  is the transmitted signal and  w(t) is the additive noise. If we knew the noise exactly, then we could subtract it from and recover the transmitted signal exactly.

The figure below shows the basic block diagram of noise.

## Signal to noise ratio.

Signal-to-noise ratio is defined as the ratio of the power of a signal (meaningful information) and the power of background noise (unwanted signal):

SNR=Power in messagepower in noise … (2)

Where P is average power. Both signal and noise power must be measured at the same and equivalent points in a system, and within the same system bandwidth.

## Additive white Gaussian noise.

Additive white Gaussian noise (AWGN) is a basic noise model used in Information theory to mimic the effect of many random processes that occur in nature. The modifiers denote specific characteristics:

- Additive because it is added to any noise that might be intrinsic to the information system.

- White refers to the idea that it has uniform power across the frequency band for the information system. It is an analogy to the color white which has uniform emissions at all frequencies in the visible spectrum.

- Gaussian because it has a normal distribution in the time domain with an average time domain value of zero. [2]

Figure 1: white noise in frequency domain

## Noise is DSB-SC

Double-sideband suppressed-carrier transmission (DSB-SC) is transmission in which frequencies produced by amplitude modulation (AM) are symmetrically spaced above and below the carrier frequency and the carrier level is reduced to the lowest practical level, ideally being completely suppressed.

The DSBSC-AM signal corresponding to m(t) is s(t) = Acm(t).cos (ωct)  and  the received signal  r(t) = s(t) + w(t) .

Since the system is linear, we can apply superposition principle, so we gets the output from the noise only

Figure 2: Power spectral density of band-pass noise

There is two types of noise SNR, which are pre-detection and post detection.

PRE-DETECTION SNR is

…(3)

While POST-DETECTION SNR is

… (4)

## Noise in SSB

SSB modulation and demodulation is similar to DSC-SC, but the only difference that we just took one side band not both. The modulated message of SSB is:

… (5)

In addition, pre detection and post detection could be calculated and the results are shown below:

… (6)

… (7)

# Procedure and discussion

## Noise in DSB-SC Modulation

Power in message after demodulation

At first, the input signal was set to sinusoidal with Vpp=5V and f=2kHz. Then, Input A was connected to the modulated signal and Input and input B was connected to the demodulated signal. The output is shown in Figure 3.

From the figure, we calculate the amplitude of the demodulated signal which is equal to 1.88, so power is A22  = 1.76 watt.  This value represents the post power of the message.

Figure 3: DSB-SC modulated signal

Power in message before demodulation.

After that, we took the output from the USB, and display it, the output is shown in Figure 4. Since the USB just has one harmonic, so it has one frequency. By taking the Amplitude of it (A = 1.15V) so power =  A22 = 0.661. But this is just in one side band, to calculate it in both bands, we just multiply it with 2  the pre power in the message equlas 1.322

Figure 4: SSB output before demodulation and before noise.

Power in noise  before demodulation

After that we insert a small AGWN with level -30db. The output in frequency domain is shown below. To calculate the power, we took the square max peak, and multiply it with the double of the bandwidth of the system which is 3.4K  the max peak was 0.02  the pre power in the noise is 0.02^2*BW*2 = 0.02^2*3.4*2 = 2.72 .

Figure 5: pre-detection noise in frequency domain with level -30DB

Power in noise after demodulation

After that, we calculate the power of the noise for post-detection (After demodulation), and this done be removing the input message and just keep the noise (we do superposition since the receiver is linear).  The power in this noise after demodulation is 0.0186^2*3.4K = 1.176. Table 1 summarize all the results that we calculated.

|  | Pre-Detection | Post-Detection |
| --- | --- | --- |
| Power in signal | 1.322 | 1.76 |
| Power in noise | 0.02^2*3.4*2 = 2.72 | 0.0186^2*3.4K = 1.176 |

Table 1: DSB-SC with -30db noise level.

To calculate SNR for pre-detection and post detection:

pre SNR=Power in messagepower in noise  = 1.322/2.72 = 0.486

post SNR=Power in messagepower in noise = 1.76/1.176 = 1.496

SNRpre/SNRPost = 1.27/1.144 = 0.324

## Noise in SSB Modulation

Power in message before demodulation

The same input as DSB-SC was excited the system (5V p-p) and 2K frequency. However, this time we used SSB with upper side band. The pre-detection power of the modulated message is calculated as is Figure 3, but now since we use SSB, we don’t multiply the power with  Amplitude of it (A = 1.15V) so power =  A22 = 0.661.

Power in noise before demodulation

The power in noise is calculated as in DSB-SC, but the only difference that the bandwidth is half in DSB (3.4K only)  from Figure 5 above, the power is equal to 0.02^2 * 3.4 = 1.36 .

Power in noise after demodulation.

As what done in DSB, by removing the input message and just keep the noise , the power of noise is the same as DSB = 0.0186^2*3.4K = 1.76.

Power in message  after demodulation.

The   same was done as the DSB; the output is shown below in Figure 6. The power of message is  A22

= 1.34^/2 = 0.8978 watt.

Figure 6: recovered message from SSB

Table 2 summarize all the results that we have calculated.

|  | Pre-Detection | Post-Detection |
| --- | --- | --- |
| Power in signal | 0.661 | 0.8978 |
| Power in noise | 0.02^2 * 3.4 = 1.36 | 1.176 |

Table 2: SSB results with -30DB noise

pre SNR=Power in messagepower in noise  = 0.661/1.36= 0.486

post SNR=Power in messagepower in noise = 0.8978 /1.76 = 0.763

SNRpre/SNRPost = 0.486/0.763= 0.63.

We can notice that SSB is better than DSB-SC, since SNRpre/SNRPost is greater.

## SSB and DCB-SC with -12db noise

The same steps was applied as above, but this time with -12db noise level. The result shown in tables below was gotten .

|  | Pre-Detection | Post-Detection |
| --- | --- | --- |
| Power in signal | 1.322 | 0.661 |
| Power in noise | 0.22^2 *3.4K*2 = 329 | 0.19535^2 * 3.4K = 129 |

Table 3: DSB with -12db noise

pre SNR=Power in messagepower in noise  = 4.0182 * 10^-3

post SNR=Power in messagepower in noise = 5.124 * 10^-3

SNRpre/SNRPost = 0.783

|  | Pre-Detection | Post-Detection |
| --- | --- | --- |
| Power in signal | 0.66 | 0.8978 |
| Power in noise | 0.22^2 *3.4 = 160 | 0.19535^2 * 3.4K = 129 |

Table 4: SSB with -12db noise

pre SNR=Power in messagepower in noise  = 4.125 * 10^-3

post SNR=Power in messagepower in noise = 6.95 * 10^-3

SNRpre/SNRPost = 0.812

We can see also that SSB is better than DSB-SC, since SNRpre/SNRPost is greater

# Conclusion

The noise exists in every communication system. So to simulate this in our lab, we have manually added AGWN to out DSP-SC and SSB modulated message. From the results from the lab, we saw that when SNR is bigger this is better, since the ratio of the message relative the noise is large. We can determine the efficient of the system for noise by calculating the formula

SNRpre/SNRPost  , As it’s becoming bigger, the receiver is better for avoiding noise. We saw in the frequency domain that AGWN following (almost) a constant voltage value. So we used this value to calculate the power of this noise (A^2 * BW).

By taking, two noise levels on the procedure, which are -30db, and -12db. We saw that SSB modulation is better in SNR than the DSB. The reason of that returns to that the bandwidth of the DSB-SC when its modulated is 2xBW, so the noise will be doubled in pre detection stage. In contract in SSB, the bandwidth of noise would be just BW of the message; therefore, the SNR will be better.

# References

[1] http://www.ece.umd.edu/~tretter/commlab/c6713slides/ch6.pdf

[2] https://en.wikipedia.org/wiki/Additive_white_Gaussian_noise

[3] Introduction to Analog & Digital Communications, Second Edit