Contents

I.	Introduction	2

Slope overload distortion:	3

Granular noise:	3

Non-return to zero [2]	4

Return to zero [3]	4

PCM Vs. DM	4

II.	Procedure and Discussion	5

Part one: prediction signals in linear delta modulation (LDM)	5

Part two: prediction in digital controlled delta modulation (DCDM).	5

Part three: pulse height in LDM	6

Part four: pulse height in DCDM	6

Part five: output signals of the LDM modulators	6

Part six: output signals of the DCDM modulator	7

Part seven: clock and DM signals in RZ format	7

Part eight: clock and DM signals in NRZ format	8

Part nine: Granular noise in LDM	8

Part ten: Granular noise in DCDM	8

Part eleven: slop overload in LDM	9

Part twelve: slope overload in DCDM	10

Part thirteen: Dynamic of LDM and DCDM	10

A) Dynamic of LDM	10

B) Dynamic of DCDM	11

PART fifteen: Coding and companding	11

A) LDM: Coding in the DM modulator	11

B) DCDM: Coding in the DM modulator.	12

C) DCDM: Compressor & DM signals	12

D) Recovery of the predicted PAM signals in DM modulator and DM demodulator.	13

Part sixteen: Demodulation	13

A)	LDM demodulation	13

B) DCDM demodulation	14

III.	Conclusion	15

IV.	References	16

# Introduction

A Delta modulation [1] (DM or Δ-modulation) is an analog-to-digital and digital-to-analog signal conversion technique used for transmission of voice information where quality is not of primary importance. DM is the simplest form of differential pulse-code modulation (DPCM) where the difference between successive samples are encoded into n-bit data streams. In delta modulation, the transmitted data are reduced to a 1-bit data stream.

To achieve high signal-to-noise ratio, delta modulation must use oversampling techniques, that is, the analog signal is sampled at a rate several times higher than the Nyquist rate.

Principle of the delta. The output signal (blue) is compared with the limits (green). The limits (green) correspond to the reference signal (red), offset by a given value. Every time the output signal reaches one of the limits, the PWM signal changes state.

Simple Implementation of a DM system.

LDM (linear delta modulation) which has an equal delta (constant) or step staircase, whereas in DCDM (digital controlled delta modulation) delta is not constant.

There are two problems associated with delta modulation that do not occur with conventional PCM: slope overload and granular noise

## Slope overload distortion:

The step size is too small for the staircase approximation to follow a steep segment of the original message signal.

The result that the approximation signal falls behind the message signal.

When the analog input signal changes at a faster rate than the DAC can maintain.

The slope of the analog signal is greater than the delta modulation can maintain and is called slope overload.

Increasing the clock frequency reduces the probability of slope overload occurring.

Another way to prevent slope overload is to increase the magnitude of the minimum step size.

## Granular noise:

When the original analog input signal has a relatively constant amplitude, the reconstructed signal has variations that were not present in the original signal.

When the step size is too large relative to the local slope characteristic of the original message signal.

The staircase approximation to hunt around a relatively flat segment of the message signal

Figure 1: slope overload and Granular noise

Illustration of quantization errors, slope overload distortion and granular noise, in delta modulation.

## Non-return to zero [2]

In telecommunication, a non-return-to-zero (NRZ) line code is a binary code in which ones are represented by one significant condition, usually a positive voltage, while zeros are represented by some other significant condition, usually a negative voltage, with no other neutral or rest condition. The pulses in NRZ have more energy than a return-to-zero (RZ) code, which also has an additional rest state beside the conditions for ones and zeros. NRZ is not inherently a self-clocking signal, so some additional synchronization technique must be used for avoiding bit slips; examples of such techniques are a run length limited constraint and a parallel synchronization signal.[3]

## Return to zero [3]

Return-to-zero (RZ or RTZ) describes a line code used in telecommunications signals in which the signal drops (returns) to zero between each pulse. This takes place even if a number of consecutive 0s or 1s occur in the signal. The signal is self-clocking. This means that a separate clock does not need to be sent alongside the signal, but suffers from using twice the bandwidth to achieve the same data-rate as compared to non-return-to-zero format.

The "zero" between each bit is a neutral or rest condition, such as a zero amplitude in pulse amplitude modulation (PAM), zero phase shift in phase-shift keying (PSK), or mid-frequency in frequency-shift keying (FSK). That "zero" condition is typically halfway between the significant condition representing a 1 bit and the other significant condition representing a 0 bit.

Although return-to-zero (RZ) contains a provision for synchronization, it still has a DC component resulting in “baseline wander” during long strings of 0 or 1 bits, just like the line code non-return-to-zero.

## PCM Vs. DM

- Signal-to-noise ratio of DM is larger than signal to-noise ratio of PCM.

- In PCM, all the bits that are used to code a sample are transmitted, whereas in DM only one bit for one sample is transmitted.

# Procedure and Discussion

## Part one: prediction signals in linear delta modulation (LDM)

Figure 2: LDM  prediction with low frequency (10 KHz)

After using LDM and took the output from prediction block, we note that the prediction is more accurate as we increase the sampling frequency. Figure 2 and Figure 3 shows the difference of prediction as we increase the sampling frequency.

Figure 3: LDM prediction at max frequency (100 KHz)

We notice that each sample, we just increased by +delta or –delta.

## Part two: prediction in digital controlled delta modulation (DCDM).

After using the DCDM modulator, We notice that we can’t see a difference from LDM. Also as I mentioned Above, As we increase the as we as we increase the sampling frequency, the prediction became better.

Figure 4: DCDM with fcl = 10KHz

Figure 5: DCDM with fcl = 100K

## Part three: pulse height in LDM

After applying a sinusoidal, 100H, 7Vpp input. And using Non-return to zero technique, with LDM. In          Figure 6, we can clearly see that as the input increase (message greater that prediction), there is a +delta which represented in logic ‘1’, but in real world, ‘1’ is represented as voltage, so we send +2V. Also we see that when the message decreased, we send logic ‘0’ which represented with -2V.

Figure 6: Pulse height in LDM fc=50 KHz

## Part four: pulse height in DCDM

After making the same setup in part three but in DCDM. In Figure 7 and  Figure 8, we can see that delta size changing as the message change. In contrast, LDM step size (delta) remain the same during the sampling.

Figure 7: DCDM after zooming

Figure 8: DCDM pulse height with Fcl =50K

## Part five: output signals of the LDM modulators

A sine modulating signal (100H, 4Vpp), the output of modulator signal (bipolar, RZ) and connections were done as required. From Figure 9, we can notice that each pulse (+delta or –delta), the output returns to zero in the half of the pulse.

Figure 9: output of LDM using Return to zero line coding

## Part six: output signals of the DCDM modulator

By using the same input signal from the previous part, but using DBCM instead of LDM. We note that they are the almost the same.

Figure 10: DCDM with bipolar RZ

## Part seven: clock and DM signals in RZ format

Here the type of DM signals is not important, so the experiment were done in LDM mode only ,the modulating signal is sinusoidal wave (200H,2Vpp),the output at the bipolar RZ. Figure 11 shows the clock and the deltas depending on the prediction and messege.

Figure 11: fcl and +deltas and -deltas are shown with RZ format.

## Part eight: clock and DM signals in NRZ format

Using the input signal above, but using NRZ output, we get this figure. We clearly can see that the output still at the same level, and it doesn’t return to zero.

Figure 12: fcl and LDM with NRZ format

## Part nine: Granular noise in LDM

The components were assembled with modulating signal square wave (50%, 200Hz, 2Vpp, ATT=0db), output was take at prediction signal X. We notice that when the input message (pulse train) is constant, LDM gives and Granular noise which is clearly shown in Figure 14. We can solve this problem by increasing the fcl as shown in Figure 13, we can see the granular noise became smaller.

Figure 13: LDM with high sampling frequency (lese granular noise)

Figure 14: LDM with low sampling frequency (more granular noise)

## Part ten: Granular noise in DCDM

Same setup in part nine but set the bridging plug in DCDM. We can see from the figures bellow that the granular noise didn’t improved. Since the DCDM can just change the step size, by it can’t control this in low frequency. In the second figure when we increase the fcl , gradual noise improved. So we can conclude that increasing fcl improve granule noise regardless of using LDM or DCDM.

Figure 15: DCDM, Granule noise, max frequency

Figure 16: DCDM with square wave input (low fcl)

## Part eleven: slop overload in LDM

By using LDM, from all the bellow figures, we can see that Slop overload in a main problem in LDM, since this modulation technique can’t change the step size (delta) when the is a huge slope in the message signal, so we just have to wait with this small step size until we pass this huge slope.

## Part twelve: slope overload in DCDM

The same setup as in the previous part but in the DCDM mode. From Figure 17, we can see that when there is a huge change in the slope, DCDM can change the step size (delta) so we pass the huge change quickly. Therefore, we can avoid slope overload problem using this technique .

Figure 17: DCDM with square wave input

## Part thirteen: Dynamic of LDM and DCDM

### A) Dynamic of LDM

In this the modulating signal was sinusoidal and the amplitude was varied until X (prediction signal) demonstrates the beginnings of slope overload. At Figure 18, we can see that the amplitude of the signal is not large, so there is no slop overload. But in Figure 19, the amplitude is high so that LDM can’t follow it with its fixed jump (delta). The Value. We noticed in the lab, that when we increase the frequency of the message, the slop overload start to occur in lower amplitude.

Figure 18

Figure 19

### B) Dynamic of DCDM

The same as above was done but using DCDM technique. Since DCDM can change the value of step, so it can stand for more amplitude until slop overload happens.

The table below summary the amplitude when slop overload starts to happen at different frequencies. Note that AMmax represents the amplitude at which overload starts, And D is is the DB value of the AMmax/ AMmin where AMmin = 20mV.

|  | LDM |  | DCDM |  |
| --- | --- | --- | --- | --- |
| Frequency (Hz) |  | AMmax | D | AMmax | D |
| 100 |  | 2.9 | 43.2 | 16.4 | 58.28 |
| 200 |  | 2.2 | 40.8 | 11.3 | 55.04 |
| 300 |  | 1.8 | 39.1 | 9.1 | 53.16 |
| 400 |  | 1.3 | 36.3 | 8 | 52.04 |
| 500 |  | 1.1 | 34.8 | 5.6 | 48.94 |
| 1000 |  | 0.6 | 29.5 | 2.2 | 40.83 |
| 2000 |  | 0.2 | 20.0 | 1.4 | 36.90 |
| 2500 |  | 0.1 | 14.0 | 0.9 | 33.06 |
| 3000 |  | 0.1 | 14.0 | 0.8 | 32.04 |
| AVG |  | - | 30.2 | - | 41.3 |

Table 1: Dynamic response of LDM and DCDM

From the table above, we can see that DCDM is much better in solving the slope overload, since it can change the step size. In addition, we can see that as we increase the frequency, the AMmax became smaller. We see that that DCDM is better with (41.3 – 30.2 = 11.1db).

## PART fifteen: Coding and companding

### A) LDM: Coding in the DM modulator

In delta modulation the coding of the differential values is performed by an edge triggered D-flip flop, functioning as sample and hold circuit .the differential values stem from the comparison of the modulating signal and the prediction signal. With each clock pulse the bit present at the D-input is shifted to the output and stored temporarily until the arrival of next clock pulse.

Figure 20

We notice from Figure 20, that when we use LDM, the step size is fixed. The zigzag signal is the +delta or –delta which have fixed voltage, and using RZ line coding.

### B) DCDM: Coding in the DM modulator.

The companding principle is derived from 0/1-distribution of the bits at the shift register. The output is shown below. We can see that the compressor used to decide to increase or decrease the step size.

### C) DCDM: Compressor & DM signals

From Figure 21, by taking the output after integrarion number and PAM modulator, we can see that the delta size is vary depend on the slop in the message signal.

Figure 21: Compressor an DM signal

### D) Recovery of the predicted PAM signals in DM modulator and DM demodulator.

In this part, we used the recovery circuit to recover the delta values which is the same in the transmitter. From the below figure, we can see that the delta in the receiver and the transmitter are equal, the reason that the receiver can create the same deltas, is that  the D-FFs in the transmitter are connected with the D-FFs in the receiver.

Figure 22: deltas in the transmitter and receiver

## Part sixteen: Demodulation

### LDM demodulation

From Figure 23 and Figure 24, we can see that when we use LDM, and when there is a huge slope, the recovered message being distorted since the modulator circuit can’t follow the message signal.  In Figure 23, the distortion is not very large since the slope is not very large, but in Figure 24,  it’s obvious that the recovered message is really distorted. After trying different frequencies, we concluded that as we increase the message frequency, the probability of distortion became bigger.

Figure 23: triangular input and output from LDM

Figure 24: square input and input 10% duty cycle

### B) DCDM demodulation

Figure 25: square input and output using DCDM

From the figure above, we can see that since we use DCDM which is adaptive DM, so the modulator can follow the message when slope overload occur because it can change the step size. Therefore, the recovered message is better than Figure 24.

# Conclusion

To conclude, Delta Modulation is a useful forms of digital pulse modulation, which is sampled with rate much higher than the Nyquist rate. The advantage of DM that it just send ‘1’ or ‘0’ every Ts sample, and it doesn’t send the whole message as some techniques, So it just use 1 bit unlike PCM which need many bits.

Delta modulation has two forms, LDM and DCDM. LDM has a fixed voltage represents logic ‘1’ which is 2V is cassylab2, and -2 for logic ‘0’. In contrast, DCDM is an adaptive DM which can change the step (delta) size according to the slope of the message. DCDM use 3-bit D-FFs and compressor to do that.

Delta modulation has two main problem. First is Granular noise, which is when there is a constant value, the transmitter keep sending ‘1’ followed by ‘0’. We can solve this problem by increasing the fs, since this will decrease the interval for the integration block, so the ‘1’ and ‘0’ would have less voltages, so the constant part of message will stay almost constant. Second problem is Slope overload, which is when there is a slope that is large than ∆/Ts, so the message can’t keep following the message. To solve this noise, we have to use DCDP since it can change ∆ size so it could follow the signal (there is some limitations). We saw from table 1 that DCDB is better with 11.1 DB.

We noticed that as the message frequency the potential of noise could happen. In addition, we see when there is a slop overload in the message and we use LDM, the recovered message is more distorted, but when using DCDM, it is much better.

# References

[1] https://en.wikipedia.org/wiki/Delta_modulation

[2] https://en.wikipedia.org/wiki/Non-return-to-zero

[3] https://en.wikipedia.org/wiki/Return-to-zero

[4] Introduction to Analog & Digital Communications, Second Edition.