# Birzeit University Faculty of Engineering and

Technology

Department of Electrical and Computer Engineering ENEE4103

Communication Laboratory

Experiment 4 Report Frequency modulation

| Prepared By: Mohammed Ahmed | 1101753 |
| --- | --- |
| Saji Alshaikh | 1110055 |
| Bassam Jaber | 1100413 |

Instructor : Dr. Wasel Ghanem TA: Eng. Nour Shanak

# Abstract

In this experiment we will simulate and configure frequency modulation circuit and in order to study and illustrates different behavior in frequency modulation circuits , also we will see the effect of FM modulation in frequency spectrum for both sinusoidal signal and square signal. Finally we will experimentally see the effect of demodulation circuit on different loop filters on different time constants.

# Table of Contents

Part I Introduction	3

  - Frequency Modulation	3

  - Carson's rule	4

  - voltage controlled Oscillator	4

Part II Procedure and discussion	5

  - Dynamic response of FM	5

  - The Characteristic of the FM Modulator	5

  - Determination of the frequency deviation	6

  - Measurements in the frequency domain of FM	7

  - Determining the carrier zero crossings by varying the frequency deviation ΔFFM. 7

  - Determine the carrier zero crossings by varying the modulating frequency fM.	8

  - FM spectrum for square wave modulation	8

  - FM demodulation for loop filter τ2	9

  - FM demodulation for loop filter τ1	9

Part III Conclusion	10

References	11

# Part I Introduction

## Frequency Modulation

Frequency modulation (FM) conveys information over a carrier wave by varying its instantaneous frequency. This is in contrast with amplitude modulation, in which the amplitude of the carrier is varied while its frequency remains constant. In analog applications, the difference between the instantaneous and the base frequency of the carrier is directly proportional to the instantaneous value of the input signal amplitude[1].

Suppose the baseband data signal (the message) to be transmitted is xm(t) and the sinusoidal carrier is  , where fc is the carrier's base frequency and Ac is the carrier's amplitude. The modulator combines the carrier with the baseband data signal to get the transmitted signal:

In this equation,    is the instantaneous frequency of the oscillator and  is the frequency deviation, which represents the maximum shift away from fc in one direction, assuming xm(t) is limited to the range ±1.

Although it may seem that this limits the frequencies in use to fc ± fΔ, this neglects the distinction between instantaneous frequency and spectral frequency. The frequency spectrum of an actual FM signal has components extending out to infinite frequency, although they become negligibly small beyond a point. As with other modulation indices, this quantity indicates by how much the modulated variable varies around its unmodulated level. It relates to the variations in the frequency of the carrier signal:

Where is the highest frequency component present in the modulating signal xm(t), and    is the Peak frequency-deviation, i.e. the maximum deviation of the instantaneous frequency from the carrier frequency. If , the modulation is called narrowband FM, and its bandwidth is approximately .

If , the modulation is called wideband FM and its bandwidth is approximately

. While wideband FM uses more bandwidth, it can improve signal-to-noise ratio significantly.

## Carson's rule

A rule of thumb, Carson's rule states that nearly all (~98%) of the power of a frequency- modulated signal lies within a bandwidth of

where  , as defined above, is the peak deviation of the instantaneous frequency   from the center carrier frequency [3].

## voltage controlled Oscillator

the VCO which is A voltage-controlled oscillator or VCO is an electronic oscillator designed to be controlled in oscillation frequency by a voltage input. The frequency of oscillation is varied by the applied DC voltage, while modulating signals may also be fed into the VCO to cause frequency modulation (FM) or phase modulation (PM); a VCO with digital pulse output may similarly have its repetition rate (FSK, PSK) or pulse width modulated (PWM)[4].

# Part II Procedure and discussion

## Dynamic response of FM

In this part of the experiment we have seen the dynamic response of frequency modulation with sinusoidal message signal with 20 v peak-peak and 1Khz frequency and using a carrier signal with frequency of 20Khz , the modulated signal is as shown in figure 2.1 .

### Figure 2.1 (Dynamic response of FM)

## The Characteristic of the FM Modulator

In this part we will study the characteristic of FM modulator , the frequency of the carrier is set to 20Khz , and we have used -10V DC-signal as an input signal to the modulator then we have increment the DC value by a step of 1 V and we measure the corresponding fVCO as the following table shows :

### Table 2.1 (characteristic carrier frequency versus the control voltage of the VCO)

| DC Value | FVCO | DC Value | FVCO | DC Value | FVCO | DC Value | FVCO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| -10 | 19.37 | -5 | 19.70 | 0 | 20.03 | 5 | 20.30 |
| -9 | 19.41 | -4 | 19.76 | 1 | 20.11 | 6 | 20.37 |
| -8 | 19.50 | -3 | 19.83 | 2 | 20.17 | 7 | 20.43 |
| -7 | 19.57 | -2 | 19.90 | 3 | 20.23 | 8 | 20.51 |
| -6 | 19.64 | -1 | 19.96 | 4 | 20.26 | 9 | 20.63 |

Figure 2.2 (Liner Characteristic of VCO)

now using the data obtained from the previous part we can determine the coefficient of FM modulator using the following relation

KFM= 1.26/20 = 0.063  Coefficient of FM modulator

## Determination of the frequency deviation

The maximum frequency deviation depends of the coefficient of the FM modulator and the amplitude of the modulating signal as the following relation shows:

From the previous relation we can measure the frequency deviation which is

## ΔFFM= 0.063 * 10 = 0.63Khz = 630 Hz

## Measurements in the frequency domain of FM

In this part we will see the frequency response of frequency modulation , first we should set the frequency of the carrier to 20Khz then we apply a message signal with 20V peak- peak with frequency of fm=300Hz and we have obtained the following figure . Then we repeat the same procedure with Fm  200Hz .

### Figure 2.3 (Frequency response of FM modulator with fm 300Hz and 200 Hz)

We can notice that the carrier amplitude has decreased when we decrease the frequency from 300 to 200 Hz.

## Determining the carrier zero crossings by varying the frequency deviation ΔFFM.

In this part we will experimentally measure the carrier zero crossing by modifying the frequency deviation values , we have used sinusoidal signal with 0Vpp and fm 100Hz and we feed it as an input to the modulator , then we changed the values of message amplitude until the carrier line disappear which is called zero crossing point.

### Figure 2.4 (FM spectrum at the first Zero Crossing)

Using the measurements obtained for the variation between frequency deviation and the carrier frequency we can calculate the modulation index as the following relation shows

Modulation index = frequency deviation / fm .

The first value for zero crossing was 3.66V and the second value obtained around 8.5Volts ,

## Determine the carrier zero crossings by varying the modulating frequency fM.

In this part we will also find the carrier zero crossing by varying the modulation frequency fm , we have used a message signal with 20v peak-peak , starting from message frequency of 3Khz , then we have decreased the frequency and notice the decay in the carrier line .

We have found that there was two zero crossing points at fm = 280Hz , and fm = 118Hz , which is corresponding to modulation index β = 2.86 , and 6.78 respectively .

## FM spectrum for square wave modulation

In this part we will see the FM spectrum of modulated square wave signal with 20 V peak- peak , with 50% duty cycle and frequency of 200Hz ,the following figure show the spectrum of modulated square wave .

### Figure 2.5 (Spectrum of square wave modulated signal )

## FM demodulation for loop filter τ2

In this part we will demodulated the FM signal using the configuration described in the experiment , we have feed the modulated signal to the FM demodulator circuit and we have set the loop filter to τ2 , the message signal amplitude is 10V peak-peak , and 500Hz frequency . the following figure shows the modulation output.

### Figure 2.6 (FM demodulated signal Vs original signal)

Due to time limit we were unable to obtain several values as required in part B which aims to measure the amplitude of demodulated signal using spectrum analyzer .

## FM demodulation for loop filter τ1

In this part we have connected and used the same configuration used in the previous part but while setting the loop filter of the FM demodulator to τ1 , the following figure show the output of demodulation using this settings .

### Figure 2.7(FM demodulation using loop filter τ1 )

# Part III Conclusion

In this experiment we have study the difference between old AM modulation techniques and frequency modulation, and we have seen in different part of this experiment the stages of frequency modulation in addition to the behavior of the modulated signal when changing the input frequency or amplitude , also we have seen and study the effect in frequency domain .

More deeply we have detect the carrier zero crossing point when we vary the value of frequency deviation and on that point we have measured the values of modulation index depending on the value obtained from carrier zero crossing points for both frequency deviation change and message amplitude .

# References

[1] Frequency Modulation :http://en.wikipedia.org/wiki/Frequency_modulation , last accessed 20-3-2015

[2] Frequency Modulation: http://fas.org/man/dod-101/navy/docs/es310/FM.htm, last accessed 20-3-2015

[3] Carson bandwidth rule : http://en.wikipedia.org/wiki/Carson_bandwidth_rule, last accessed 20-3-2015

[4] Voltage controlled oscillator: http://en.wikipedia.org/wiki/Voltage- controlled_oscillator ,last accessed 20-3-2015