Faculty of Engineering and Technology

Electrical and Computer Engineering Department

Communication Laboratory

ENEE4103

Experiment#2  Report

____________________________________________

Single Sideband AM (SSB) and Ring Modulator

____________________________________________

Prepared by:

Reem Mohammad		ID: 1121412

Sondos Dahbour		       ID: 1120260

Haya Risheg                             ID: 1120774

Instructor: Dr. Mohammad Wasfi Al-Kafri

TA :Eng. Nour shanak

# Abstract

In This experiment we carried out the single side band modulation technique with two approaches , the first approach was with residual (SSB rc) carrier and the second approach was with suppressed carrier(SSB sc) to understand SSB modulation . also , in this experiment we carried out demodulation for SSB rc and the spectrum for the  two approaches . At the last,  we carried out the ring modulator and illustrated the behavior in frequency domain.

Aim :

to understand the  SSB modulation and demodulation with Residual Carrier (SSB rc) and suppressed carrier (SSB sc) and differentiate between them . Also, to  recognize the spectrum for both to differentiate between them .Also , to understand the implement of the ring modulator.

Table of Contents

Theory  ................................................................................................................1

Procedure and Discussion ...................................................................................4

2.1 SSB with Residual Carrier (SSBRC) Modulation ...............................................4

2.2 SSB with Suppressed Carrier (SSBSC) Modulation ...........................................7

2.3 Spectrum of the SSBRC ..................................................................................8

2.4 Spectrum of the SSBSC ...................................................................................9

2.5 SSBRC demodulation .....................................................................................10

2.6 SSBSC demodulation ......................................................................................11

2.7 The Ring Modulator ........................................................................................12

2.8 Spectrum at the output of the ring modulator .................................................13

Conclusion ............................................................................................................. 14

Questions ...............................................................................................................15

References ..............................................................................................................15

- Theory:

Single-SideBand modulation (SSB) or Single-SideBand Suppressed-Carrier (SSB-SC) is a refinement of amplitude modulation which uses transmitter power and bandwidth more efficiently. Single-sideband modulation avoids this bandwidth doubling, and the power wasted on a carrier, at the cost of increased device complexity and more difficult tuning at the receiver .[1]

Single Sideband (SSB) Modulation is similar to DSB modulation, but instead of using the whole spectrum it uses a filter to select either the lower or upper sideband. The selection of the lower or upper sideband results in the lower sideband (LSB) or upper sideband (USB) modulation, respectively. There are two approaches to eliminating one of the sidebands, one is the filter method and the other is the phasing method. The process of selective filtering of the upper or lower sideband is difficult due to the stringent filters required, especially if there's signal content close to DC

SSB modulation requires the shifting of the message signal to another center frequency without creating pairs of frequency components X(f-fo) and X(f+fo) as in the case of the DSB modulation, i.e., avoiding the need to filter either the upper or lower sideband. This can be done by using a Hilbert Transformer.[2]

Single-sideband has the mathematical form of quadrature amplitude modulation (QAM) in the special case where one of the baseband waveforms is derived from the other, instead of being independent messages:

where is the message, is its Hilbert transform, and is the radio carrier frequency.

Page|1

One method of producing an SSB signal is to remove one of the sidebands via filtering, leaving only either the upper sideband (USB), the sideband with the higher frequency, or less commonly the lower sideband (LSB), the sideband with the lower frequency. Most often, the carrier is reduced or removed entirely (suppressed), being referred to in full as single sideband suppressed carrier (SSBSC).[1]

Assuming both sidebands are symmetric, which is the case for a normal AM signal, no information is lost in the process. Since the final RF amplification is now concentrated in a single sideband, the effective power output is greater than in normal AM (the carrier and redundant sideband account for well over half of the power output of an AM transmitter).[1]

Page|2

Forms of SSB, single sideband modulation

There is a number of different formats of single sideband modulation that are used:

- Single sideband suppressed carrier, SSBSC :   SSBSC is the form of single sideband modulation that is most widely used for communications applications on the HF portion of the radio spectrum. There is only one sideband - the other sideband and the carrier are both removed. Having no carrier, this needs to be re-inserted within the receiver. Any slight differences in carrier re-insertion frequency give rise to changes in pitch of the audio. However it gives the most efficient spectrum and power usage of any single sideband modulation format.

- Single sideband reduced carrier :   This form of single sideband modulation removes one sideband, but retains a small amount of carrier. The reduced carrier element can then be used to lock a local oscillator for carrier re-insertion and the demodulation of the sideband with the correct audio pitch. Naturally the power efficiency of this form of single sideband modulation is not as high as single sideband suppressed carrier.[3]

Ring modulation :

is a signal-processing function in electronics, an implementation of amplitude modulation or frequency mixing, performed by multiplying two signals, where one is typically a sine wave or another simple waveform. It is referred to as "ring" modulation because the analog circuit of diodes originally used to implement this technique took the shape of a ring.[1] This circuit is similar to a bridge rectifier, except that instead of the diodes facing "left" or "right", they go "clockwise" or "anti-clockwise".[4]

Ring modulator                                                                       Page|3

# 2) Procedure and Discussion

The procedure for this experiment consisted of eight parts; each of them will be discussed separately in the following  pages.

2.1: SSB with Residual carrier(SSB rc) Modulation

Figure 1

The circuit of SSB modulation was connected as indicated in the experiment ; the output of the function generator was connected directly to the AF input of the modulator M2 . then the function generator was sit to sinusoidal Am=1.5 volt and fm=2 Khz.  The output signal was displayed  As shown below on figure 2.1.1 for modulating and modulated signals.

Figure 2.1.1 :Single side band Residual carrier modulation

Then , the AF signal was shifted to the upper or to the lower envelop detector  to make sure that envelop matches the modulating signal Sm(t) as on the Normal AM modulation . as shown below on figure 2.1.2

Figure 2.1.2: Shift the AF signal                                                          Page|4

Then , the frequency fm and the amplitude Am of the modulating signal was varied . as shown below on  Figure2.1.3 && Figure2.1.4

Figure2.1.3:   Am =3 && fm=3

Figure 2.1.4: Am=6 && fm=10

We observed from the above figures , as we varied Am and  fm  the modulation index was varied .On figure 2.1.3 modulation index (m) <1 which called it under modulation. But on the figure 2.1.4 modulation index >1 (over modulation).

Page|5

The last step was to switch off the modulating signal and  the un-attenuated carrier amplitude Ac   at the input of the channel was measured  , after that the amplitude Arc of the attenuated carrier at the output of the channel filter was measured . This is shown below on Figure 2.1.5 .

Figure 2.1.5: Un-attenuated and attenuated carrier.

- Ac  was measured , which equals   1.37/2=0.7

- Arc  was measured , which equals 0.84/2=0.42

- Then the ratio K was calculated as K= Arc/Ac  = 0.42/0.7 = 0.6

Carrier suppression t in dB according to the equation t = 20 log (m / 2k )

m (modulation index) = (1.5-0.7)/ (1.5+0.7)= 0.4 <1 (under modulation)

t = 20 log (0.4 / 0.6 * 2) = -9.5 dB

Page|6

2.2: SSB with Suppressed Carrier (SSBSC) Modulation

The circuit of SSB modulation was connected as indicated in the experiment ; the output of the function generator was connected directly to the AF input of the modulator M2 .  the function generator was set to sinusoidal Am=2 volt and fm=2 Khz .Then the Cassy Lab 2 example Diagram 4.1-2.labx was loaded .   The output signal was displayed  As shown below on figure 2.2.1.

Note: Don’t forget that the carrier was off on this part (carrier suppressed).

Figure 2.2.1:Single side band suppressed carrier

Page|7

2.3 : Spectrum of the SSBRC

The circuit of SSB modulation was connected as on part one(2.1) with the toggle switch was set to CARRIER ON . after that, the sinusoidal  modulating signal was set to Am=2 volt and fm= 1 KHZ and then it was fed into the input LP filter of the CF transmitter. the spectrum of single side band residual carrier was appeared as shown below on figure 2.3.1 in the range of approximately 15Khz up to 25Khz .

figure 2.3.1: Spectrum of the SSBrc

from the above figure , bandwidth equal to the 2KHZ (double to the frequency of the modulating signal).

LSB on 19KHZ (fc-fm) and USB on 20KHZ (fc+fm)……. So bandwidth = 2*fm.

Page|8

2.4:  Spectrum of the SSBSC :

The same was done here as on the part three but the Carrier on this part wasn’t  sended on the transmitter which is increase the power efficiency . the result was shown below on Figure 2.4.1.

Figure 2.4.1: Spectrum of the SSBsc

the bandwidth of this case is equal to 2Khz which is the double of the  frequency of the message signal .

Note that :

Bandwidth = USB frequency – LSB frequency = (fc+fm) – (fc-fm) = 2*fm= 2*1KHZ = 2KHZ

Page|9

2.5 :  SSBRC demodulation

The circuit of SSB demodulation was connected as indicated in the experiment.

The carrier signal which was 20 KHZ was fed into the RF input of the demodulator D2 , the message signal was have an Am=2 volt and fm= 2 KHZ. Then the CASSY LAB 2 Diagram 4.3-1.Labx was loaded .The result was shown below on Figure 2.5.1 .

Figure 2.5.1: SSB-RC demodulation signal Vs. Original Message

From above , we noticed that the modulating and the demodulating signal have the same frequency and phase . But the amplitude of the demodulating signal(red) is not the same as the amplitude of the modulating signal.

Page|10

2.6 :  SSBSC demodulation

In this part the connecting between the CF transmitter and the demodulator was removed to demodulate using the recovered auxiliary carrier from the PLL circuit to recover the carrier.

The toggle switch was CARRIER OFF. Then the CASSY LAB2 Diagram 4.3-2.Labx was loaded

The output was displayed below on Figure 2.6.1 .

Figure 2.6.1: SSB-SC demodulation signal Vs. Original Message using PLL circuit

We noticed from above that we have the same result which we have it on part five, which mean that we can use PLL circuit to recover the carrier on the receiver .

Page|11

2.7 :  The Ring Modulator

The circuit of the ring was connected as in the experiment .the toggle switch was set to the CARRIER OFF , also the square wave was fed into the RF input of the modulator using the bridging plug and the sinusoidal signal was fed as the modulating signal into the AF input of the modulator with fm= 2KHZ and am=2 volt. Then the CASSY Lab2 example diagram 5.1-2.labx , the output was displayed as below on figure 2.7.1.

figure 2.7.1 :Ring modulation (using square-wave carrier)

As we can see the modulation worked with square carrier because we did not feed any filter to the input signal , also you can notice that the carrier signal is not smooth on the edges because of the square wave harmonics.

Page|12

The next step was to set the toggle switch to CARRIER ON  and the CASSY Lab2 Diagram 5.1-2.labx was loaded to have the figure 2.7.2 as shown below.

2.8 :  Spectrum at the output of the ring modulator

In this part , the spectrum of the output of the modulator M2 was record in the range 0.5 KHZ to 150 KHZ. The CASSY Lab2 example Diagram 5.1-3.labx was loaded to have Figure 2.8.1  below.

Figure 2.8.1 :Spectrum of ring Modulator

From above , we noticed that as we go to infinity on X-axis the spectrum will go to Zero, also we noticed that we have the same spectrum which we have it on SSB sc around Fc .

page|13

3) Conclusion:

After this experiment , we understood very well the modulation and demodulation structure for SSB , we make sure that SSB uses the transmitter power and bandwidth more efficiently. Modulation can be achieved by a filtering method and demodulation can be achieved by an envelope detector circuit or by coherent demodulation.Now we can differentiate between SSB rc (which retains a small amount of carrier to make sure that carrier recover correctly on the receiver) and SSB sc(which There is only one sideband - the other sideband and the carrier are both removed) .

We conclude that  , the  SSB sc gives the most efficient spectrum and power usage of any single sideband modulation format , but  the SSB rc  the power efficiency of it  is not as high as single sideband suppressed carrier.

At the last part , we understood the implementation of the Ring modulator to have a modulated signal and then we have a spectrum for it ; we noticed as we go to infinity on X- axis , the spectrum will go to zero.

At the end of the experiment, we achieved our aim. Moreover, we were introduced to the new lab kits and we learned how to use the CASSY software which was very interesting but at the same time we faced many problems on dealing with the new kits ; it was very sensitive.

Page|14

4)Questions :

Q.1. In radio links the carrier is normally attenuated to 5%...10%. What advantages does this have compared to transmission with 100% carrier amplitude? Why isn't the carrier completely suppressed?

It appears if its completely suppressed but the attenuation ratio forbids the overlap of different signals.

Q.2. Which methods of carrier suppression are there?

Single side band suppressed carrier (SSB-SC) and double side band suppressed carrier (DSB-SC).

Q.3. Which demodulation method is used for AM with suppressed carrier?

Envelop detector and coherent demodulator, but coherent demodulator is better when the received signal is corrupted with additive noise.

5)References:

[1] https://en.wikipedia.org/?title=Single-sideband_modulation

[2]http://www.mathworks.com/help/signal/examples/single-sideband-modulation-via-the-hilbert-transform.html

[3]http://www.radio-electronics.com/info/rf-technology-design/am-amplitude-modulation/single-sideband-ssb-modulation.php

[4] https://en.wikipedia.org/?title=Ring_modulation

Page|15