BIRZEIT UNIVERSITY

Electrical & Computer Engineering Department

## ENEE 4101

COMMUNICATIONS LAB

REPORT  No. 2

AM and DSBsc Modulation and Demodulation

Prepared by:

Student’s Name.    DINA ASSI             Student’s No.:  1120872

Partners:

Student’s Name.    hala nahal           Student’s No.:  1111512

Student’s Name.    dana abu libdeh      Student’s No.: 1110375

Instuctor: 		Dr.Wasel Ghanem                         Section No.:    (“1 “)

# Assistant:		 Eng. Noor Shannak .

Date:	   4\ 3\ 2015

ABSTRACT:

The main aim of this experiment is to implement Amplitude Modulation “AM” which is an analog way to transfer information (message signals) over the air, using the Amplitude part of the carrier signal to keep the information away from the outer noise sources, we will see the modulated signal in time domain and frequency domain. Also we will do DSB and see its modulated signal in time domain and frequency domain.

Theory:

The purpose of a communication system is to transmit the information signals through medium of communication channel, and these signals are at low frequencies. So to transmit these signals we used a carrier signal and that signal is at high frequency. The carrier signal has three parameters, the first one is the amplitude, the second is the frequency, and the phase of the carrier signal .

Detection means the extraction of the information (the detecting of the modulating wav) from the modulated wave.Simple method of detection and by far the most common one is Envelope Detection. This method is based on the fact that the information is contained in the envelope of the AM waveform. The principle of Envelope Detector is simply detecting the original message races form the modulated signal and filter it away.

Modulation is the process of varying a periodic waveform, i.e. a tone, in order to use that signal to convey a message..Normally a high-frequency sinusoid waveform is used as carrier signal. The three key parameters of a sine wave are its amplitude, its phase and its frequency, all of which can be modified in accordance with a low frequency information signal to obtain the modulated signal.

Device that performs modulation is known as a modulator and a device that performs the inverse operation of modulation is known as a demodulator, a device that can do both operations is a modem (short for "Modulate - DEModulate").

Amplitude modulation is a process of translating information signal from low band frequency to high band frequency. Information signal is usually of frequency, so it cannot travel far. It needs carrier signal of higher frequency for long distance destination. Figure 1 shows a basic block diagram of a modulator. The inputs are carrier and information (modulating) signals while the output is called modulated signal.

Fig1

In Amplitude Modulated (AM) signal, amplitude of the carrier signal varies with the information signal.

If message signal m (t) = Am cos(ωmt)

And the carrier     c  (t) = AC cos(ωct)

So the modulated wave should be S(t) = AC(1+(Am/AC )m(t) )cos(ωct)

m is the modulation coefficient =Am/AC

The modulation coefficient m shows the relation between the original carrier amplitude and the signal wave amplitude i.e a wave with m = 0.5 is called 50% modulated signal

m should be < 1 and > 0 so a true Am can be received.

For values more then 1 the envelope of the wave doesn’t resemble the original signal wave form and that called excess modulation where the information is distorted.

For m = 1 the power changes form twice Ac to 0

So a value for m between less than 1 and more than zero will be good.

Fig.2

Now to achieve the S(t) Am modulated signal form

S(t) = AC(1+(Am/AC ) m(t) ) cos(ωct)

A simple look to the equation tells that we need a multiplier to form the modulation

Here the multiplier should be an analog one so a FET transistor should do the job.

Finally the analysis of the bandwidth should be as follows

VAM(t)= VC cos ωct +mVC cosωct cosωmt

VAM(t)= VC cos ωct + mVC (1/2 cos(ωc+ωm)t + ½ cos (ωc-ωm)t)

VAM(t)= 0.5mVC cos (ωc-  ωm ) + VC cos(ωct) + 0.5mVC cos (ωc+ωm)t)

The Three frequencies above (ωc-  ωm), ( ωc) and ( ωc+ ωm) acts as the Low side band , Carrier and Upper side band respectively

So the each of the side bands contains the original information at shifted frequency Therefore the AM is the process which shifting the information signal to two different frequencies equally spaced around the carrier frequency, and that holds for any wave signal not only the pure sine ones.

Finally it’s worth to note that the bandwidth

Required to transmit an AM signal is exactly

Twice the bandwidth of the modulation signal

So put big question mark about this form of

Modulation.

Double-sideband suppressed-carrier transmission (DSB-SC) is transmission in which frequencies produced by amplitude modulation (AM) are symmetrically spaced above and below the carrier frequency and the carrier level is reduced to the lowest practical level, ideally being completely suppressed.

In the DSB-SC modulation, unlike in AM, the wave carrier is not transmitted; thus, much of the power is distributed between the sidebands, which implies an increase of the cover in DSB-SC, compared to AM, for the same power used.

DSB-SC transmission is a special case of double-sideband reduced carrier transmission

DSB-SC is basically an amplitude modulation wave without the carrier, therefore reducing power waste, giving it a 50% efficiency. This is an increase compared to normal AM transmission (DSB), which has a maximum efficiency of 33.333%, since 2/3 of the power is in the carrier which carries no intelligence, and each sideband carries the same information. Single Side Band (SSB) Suppressed Carrier is 100% efficient.

Spectrum plot of an DSB-SC signal(fig.b)

Fig.b

DSB-SC is generated by a mixer(fig.a)

Fig.a

procedure:

# Part One: AM Modulation

- AM modulation has been connected with sinusoidal message signal where AM = 2 V and fM = 2 kHz. And toggle switch was set to CARRIER ON setting.

- The modulated signal was observed after varying the frequency Fm and the amplitude Am of the modulating signal of the frequency generator which results the following figures (3&4&5).

Fig3(over modulated signal μ  more than 1)

Over modulated signal was generated after increasing the value of amplitude of the message signal.Over modulation results in spurious emissions by the modulated carrier, and distortion of the recovered modulating signal. This means that the envelope of the output waveform is distorted.

Fig.4(under modulated signal μ>1)

In this case , the signal can be recovered and the original signal can be retrieved without distortion{demodulate}

Fig.5(100% modulated signal μ=1)

In this case the amplitude Am=4.4v

The modulation depth was determined by reducing the amplitude of the message signal  to 2v and by plotting XY modus regardless the distortion

where:

o D: Peak-to-peak value of the maximum of the AM signal

o d: Peak-to-peak value of the minimum of the  Am signal

# Part Two: DSBSC Modulation

- The experiment setup was kept as in part one but  the toggle switch was set

- to CARRIER OFF. where AM = 2 V and fM = 2 kHz.

It is DSB-SC with d=D (fig.6)
                fig.6

It was observed that the low pass filter is used to retrieve the original signal instead of the envelope detector

# Part Three: Spectrum of AM Modulation

- the experiment  setup was kept as in part one but  we have set the toggle switch

- to CARRIER ON. where AM = 2 V and fM = 2 kHz.

the modulating signal have been fed into the input LP filter of the CF transmitter.  And the AM spectrum has been measured in the range from approx. 15 kHz up to 25 kHz.

Am modulation without low pass filter

Am modulation with low pass filter

Spectrum without low pass filter

Bandwidth = 23-20 = 3 KHz

Modulation depth m= 2.04/0.23 = 8.86

Spectrum with law pass filter