In many circuits there is a need to electronically translate analog signals into digital (binary) quantities analog to digital (ADC) digital  and vice versa  digital to  analog (DAC) converters

Analog to Digital Convertor (ADC) : 
An ADC inputs an analog electrical signal such as voltage or current and outputs a binary number. In block diagram form, it can be represented like this:

. The ADC process utilizes sampling and quantization of the continuous analog utilizes sampling and quantization of the continuous analog signal.

This operation is shown bellow

ADC sampling occurs at a uniform rate  (the sampling rate sampling rate) and has a continuous amplitude.

The continuous amplitude sample is then quantized to n bits or resolution for the bits or resolution for the full scale full scale input or input or 2n levels

The analog signal The analog signal x(t) which is continuously, uniformly sampled is represented by

Xst= xtk=-∞∞δ(t-kTs)

Multiplication in the temporal domain is convolution in the frequency domain and the frequency domain representation is

Xsf= Xf*F[k=-∞∞δ(t-kTs)]

Xsf= Xf*[ fs*k=-∞∞δ(f-kfs)]

Xsf= fs*k=-∞∞X(f-kfs)]

Condition : sampling frequency fS should be  greater than twice  the highest frequency fmax fS > 2 fmax

In one Picture The whole Analog to digital Convertor Process

Result will be like figure bellow [ideal Sampling ]

Digital-to-Analog Converter (DAC ) :

DAC An electronic device, often an integrated circuit, that converts a digital number into a corresponding analog voltage or current.

• Relation between analog signal and digital equivalent

Va= i=1nbi 2i x Vref

Types of DAC Circuits

1. Resistor-string

2. N-Bit Binary Weighted Resistor

3. R-2R Ladder

Figure Above shows a 4bit DAC

Other input bits on the left side could be added  , until n inputs are completed.
 The maximum input count is 2n -1 (all the n bits set to 1.) The circuit generates an output voltage from zero to    Vout=Vref 10 2n -12n

Result of the whole DAC operation is an Analog continuous Signal Generated back from digital pulses

Faculty of Information Technology

Electrical Computer Engineer  Department

Communication Lab ENEE 4103

PreLab – Ex # 4 [ ADC ,DAC]

- Students name : Sondos Dahbour

- ID # : 1120260