Faculty of Engineering And Technology

Electrical Engineering Department

Communications Lab - ENEE4103

Experiment #1

AM and DSBsc Modulation and Demodulation

Prelab

Student: 
Khaled Qarout 	1110721

Section 1

- Write the mathematical expression for the demodulated signal.

M (t) = Ac |1 +μ cos (ωmt)|

- Use MATLAB command and M files to draw the demodulated signal for the following three cases, discuss the result in each part.

a. Ac=16v, modulation index=0.22, modulating signal frequency= 800Hz.

Ac = 16;

fm = 800;

fc=8000;

Mi = 0.22;

Mi2 = 1;

Mi3 = 1.85;

t = 0:0.00001:0.01;

modulatedSignal1 = Ac * (1+Mi*cos(2*pi*fm*t)).*cos(2*pi*fc*t); %S(t)	demodulatedSignal1 = Ac*abs(1+Mi*cos(2*fm*pi*t)); %M(t)

figure;

plot(t,modulatedSignal1);

title('Modulation: Mi = 0.22');

figure;

plot(t,demodulatedSignal1);

title('Demodulation: Mi = 0.22');

μ < 1, the modulation is under modulation. However, the signal was recovered correctly in the demodulation

b. Ac=16v, modulation index=1, modulating signal frequency= 800Hz.

Ac = 16;

fm = 800;

fc=8000;

Mi2 = 1;

t = 0:0.00001:0.01;

modulatedSignal2 = Ac * (1+Mi2*cos(2*pi*fm*t)).*cos(2*pi*fc*t); %S(t)

demodulatedSignal2 = Ac*abs(1+Mi2*cos(2*fm*pi*t)); %M(t)

figure;

plot(t,modulatedSignal2);

title('Modulation: Mi = 1');

figure;

plot(t,demodulatedSignal2);

title('Demodulation: Mi = 1');

μ = 1, the modulation is proper modulation. As well as recovering the signal in demodulation.

c. Ac=16v, modulation index=1.85, modulating signal frequency=800Hz.

Ac = 16;

fm = 800;

fc=8000;

Mi3 = 1.85;

t = 0:0.00001:0.01;

modulatedSignal3 = Ac * (1+Mi3*cos(2*pi*fm*t)).*cos(2*pi*fc*t); %S(t)

demodulatedSignal3 = Ac*abs(1+Mi3*cos(2*fm*pi*t)); %M(t)

figure;

plot(t,modulatedSignal3);

title('Modulation: Mi = 1.85');

figure;

plot(t,demodulatedSignal3);

title('Demodulation: Mi = 1.85');

μ > 1, the modulation is over modulation. The original signal was not recovered.