Computer System engineering Communications laborartory

Pulse Code Modulation (PCM)

Experiment #6 & Experiment #7

Prepared by: Khaled Qarout 1110721 Yara Al Shafie 112 Elias Abu Hadba 1100537

11/19/2015

Instructor: Nofal Nofal Section 1

# Abstract

In this lab report, a brief introduction about one kind of pulse modulation is presented,  pulse code modulation. Further explanation is presented in the procedure where the results of the two experiments are explained and discussed.

Table of Contents

Abstract	0

1.	Introduction	1

1.1	Pulse Code Modulation (PCM)	1

2.	Procedure and Results	1

2.1 linear quantization	1

2.2 Non-linear quantization	2

2.3 compressor/ expander characteristic	2

2.4 Resolution of quantizer	3

2.5 Encoding	3

2.6 PCM Transmission	5

2.7 Quantization noise	6

2.8 Quantization noise variations	7

2.9 Difference pulse code modulation (DPCM)	8

3.	Conclusion	10

References	10

# Introduction

## Pulse Code Modulation (PCM)

Pulse code modulation (PCM) is a digital scheme for transmitting analog data. The signals in PCM are binary; that is, there are only two possible states, represented by logic 1 (high) and logic 0 (low). To obtain PCM the instantaneous amplitude of the analog signal at each sampling is rounded off to the nearest of several specific, predetermined levels. This process is called quantization. The number of levels is always a power of 2 [1].

Amplitude quantization is defined as the process of transforming the sample amplitude m(nTs) of a message signal m(t) at time t=nTs into a discrete amplitude v(nTs) taken from a finite set of possible amplitudes. Quantization is an important stage in forming the PCM signal where the output of the sampling process is quantized to provide a new representation that is discrete in both time and amplitude. Quantization can be of a uniform or nonuniform type. In a uniform quantizer, the representation levels are uniformly spaced; otherwise, the quantizer is nonuniform. In system that uses uniform quantizer, the quantization noise is the same for all signal magnitude. Therefore, in uniform quantization, the SNR is worse at low level signals than for high level signals.

At the destination (receiver end) of the communications circuit, a pulse code demodulator converts the binary numbers back into pulses having the same quantum levels as those in the modulator. These pulses are further processed to restore the original analog waveform [2].

# Procedure and Results

## 2.1 linear quantization

In this part liner quantization of message signal will be performed, the mode of both modulation and demodulation was set to PCM, and all 8 bits of the quantizer were activated in order to obtain high precision in quantization process.

Figure 2.1 (Modulator input voltage Vs. demodulator output)

The output is plotted in cassy to see the effect of linear quantization as shown in the previous figure, the linear relation between the input and the output voltage states that the same quantizer is used in both modulation and demodulation so the relation is linear.

## 2.2 Non-linear quantization

In this part both PCM modulator and PCM demodulator are in the mode non-linear quantization.

Figure 2.2 Non-linear quantization input Vs. output voltage

As we see the linear relations between input and output since both modulator and demodulator are in the mode nonlinear.

## 2.3 compressor/ expander characteristic

In this part one of both modulator and demodulator is in the mode linear and the other is in the mood nonlinear. So the relation between the input voltage and output voltage will describe the compressor or expander characteristic as shown in figure 2.3.

Figure 2.3 (Compressor/Expander characteristic)

## 2.4 Resolution of quantizer

This part the same as before but here there is a reduction from 8 to 5 bits.

Figure 2.4 (Non-linear quantization with 5 bits resolution)

The relation is approximately linear but the step shown in the figure due to reduction of resolution of quantization stage, further more we can see the non-linear effect by noticing the step size of each level.

## 2.5 Encoding

Figure 2.5.1

Figure 2.5.2

The circuit was connected as shown in the experiment and both modulator and demodulator were set to PCM, and all bits of quantizer were set ON, PCM modulator and demodulator was set to the mode linear quantization.

We start varying the voltage from -10 volts to 10 volts in the input and we can see that in demodulation part that leds on the demodulator part are all off while we start increasing the voltage the upper bits starts. We take the output in demodulation when two input (positive and negative) were as follow (2.33 -2.33) to get the binary sequence (10011101-00011110).

## 2.6 PCM Transmission

In this part PCM modulator and demodulator is in the mode of linear quantization ,all bits in the PCM are active, two input signals are generated (sinusoidal and triangle signals ), to see the output on demodulator channel 2 as below in figure 2.6.1,.2,.3

Figure 2.6.1

Figure 2.6.2

Figure 2.6.3

## 2.7 Quantization noise

Just one input signal was generated (triangle signal with 12 Vp-p) ,both CH1 and CH2 were connected to avoid time gaps at the output of the PCM modulator ,the mode of PCM modulator and demodulator were set to linear quantization all bits were active in the PCM modulator .

Figure 2.7.1 input Vs output modulator

Input Vs. output modulator with resolution of 5 bits, FM =300hertz

## 2.8 Quantization noise variations

This part the same as part 1 but with a more complicated structure of the quantization noise and with a sinusoidal modulating signal.

Figure 2.8.1 both PCM modulator and demodulator in linear mode

Figure 2.8.2 both pcm modulator and demodulator in nonlinear mode

## 2.9 Difference pulse code modulation (DPCM)

Ch1 and ch2 of the pam modulator were connected to time gaps at the output of PCM modulator. Modulating signal is a triangle with FM=30h, pcm modulator and demodulator are in linear mode.

Figure 2.9.1 modulating signal (pam input)

Figure 2.9.2 modulating signal after predictor

Figure 2.9.3 after pcm

Figure 2.9.4 before dpcm

Figure 2.9.5 pam output of the dpcm demodulator

# Conclusion

PCM is a form of pulse modulation. Other methods include varying the durations (or widths), the frequencies, the positions or the amplitude (PAM) of the individual pulses in a sequence.  PCM offers advantages over other modulation methods in its resistance to noise and its ability to be processed digitally. PCM is particularly resistant to noise added once it has been modulated in the communication channel.

# References

[1] http://site.iugaza.edu.ps/mabufoul/files/2010/02/Exp7.pdf  accessed on November 18th, 2015

[2] http://searchnetworking.techtarget.com/definition/pulse-code-modulation-PCM  accessed on November 18th, 2015