بسم الله الرحمن الرحيم

Computer System Engineering Department

ENCS 211

DIGITAL ELECTRONICS AND COMPUTER ORGANIZATION LAB

Report For Experiment NO.10

Introduction to DEBUG Program.

- Students name & ID: Hanadi Mzahim, 1071152 & Eman Arar, 1070720.

- Instructor Name:  MR. Mohammad Abu-Ayyash.

- Teacher Assistant: ENG. Anas Abd-elrazeq.

- Date: 14-4-2010.

- Section: 2.

- Part 1: Immediate Operands.

- Activity 1.1:

We firstly opened the CMD screen and we activated the debug command, and we wrote the following instructions written in assembly language  at the offset memory locations 100h ,by typing A100 then  :

MOV AX, 2864

ADD AX, 3749

MOV BX, AX

SUB BX, 2805

NOP

- Activity 1.2:

Then we used the command U to unassembled the instructions and we get the machine code that is represented in table (1) :

| Assembly code | Machine code |
| --- | --- |
| MOV AX, 2864 | B8 64 28 |
| ADD AX, 3749 | 05 49 37 |
| MOV BX, AX | 89 C3 |
| SUB BX, 2805 | 81 EB 05 28 |
| NOP | 90 |

Table (1)

We notice that we get the machine code for each instruction.

Activity 1.3: by this command we also can get the number of bytes needed for each instruction, which is shown in table (2).

| Assembly code | Machine code |
| --- | --- |
| MOV AX, 2864 | 3 |
| ADD AX, 3749 | 3 |
| MOV BX, AX | 2 |
| SUB BX, 2805 | 4 |
| NOP | 1 |

Table (2)

As we note the number of bytes is differ from one instruction to another.

- Activity (1.4):

The immediate data value (2864) stored in location 101H, by situate:

64 in location 101 H

28 in location 102 H

This result is dependent that the system is little Indian, so the lowest byte is put in the first location, and the highest one on the next one …

- Activity 1.5:

Then we used the command R, which we used to determine the contents  of the registers as shown in table (3) below:

| Register | Content |
| --- | --- |
| CS | 1395 |
| IP | 0100 |
| AX | 0000 |
| BX | 0000 |

Table (3)

We note from the results we get in the previous table, since we didn’t execute the instructions yet, the value of code segment represent the location of the code, the instruction pointer (IP) represent where we want to be the code segment which is location 100h that we wrote our instructions from, the other registers (AX, BX) have no initial values yet…

- Activity 1.6:

The predictions of the contents of CS, IP, AX and BX:

| Register | MOV AX, 2864 | ADD AX, 3749 | MOV BX, AX | SUB BX, 2805 |
| --- | --- | --- | --- | --- |
| CS | 1395 | 1395 | 1395 | 1395 |
| IP | 0103 | 0106 | 0108 | 010C |
| AX | 2864 | 5FAD | 5FAD | 5FAD |
| BX | 0000 | 0000 | 5FAD | 37A8 |

Table (4)

We note from the results above that the expected  value of the instruction pointer is changing or increasing by the same number of bytes for each instruction as shown in table (2) , and also the  expected values of AX , BX is changing according to the instructions will  be  executed .

- Activity 1.7:

After executing the instructions by command T, the real contents of CS, IP, AX and BX are as follows:

| Register | MOV AX, 2864 | ADD AX, 3749 | MOV BX, AX | SUB BX, 2805 |
| --- | --- | --- | --- | --- |
| CS | 1395 | 1395 | 1395 | 1395 |
| IP | 0103 | 0106 | 0108 | 010C |
| AX | 2864 | 5FAD | 5FAD | 5FAD |
| BX | 0000 | 0000 | 5FAD | 37A8 |

Table (5)

We note that the real is identical to the predict values, to the reasons we mentioned in the last activity …

- Activity 1.8:

The content of IP changes after each instruction is executed so that it points to the next instruction to be executed; the amount of change in the IP indicates how many bytes the instruction took to be represented in machine code.

- Activity 1.9:

The offset address of the second MOV instruction is 106 H.

Its logical address = 1395: 0106.

Its physical address = 1395 * 10 H + 0106 H = 13A56 H

- Part 2: Memory addressing.

- Activity 2.1:

We enter the following data at the offset memory location 200h using DEBUG command E.

E DS: 200 1B 9F

E DS: 202 36 4A 00 00

E DS: 206 2A 2A 2A

- Activity 2.2:

Using DEBUG command E, the following program instructions in machine code is entered at the offset memory location 100h.

E CS: 100 A1 00 02

E CS: 103 8B 1E 02 02

E CS: 107 01 C3

E CS: 109 89 1E 04 02

E CS: 10D 90

- Activity 2.3:

We use DEBUG command U (unassemble) to know the assembly code corresponding to each machine code Instructions (in activity 2.2). The results are in the table below:

| Assembly code | Machine code |
| --- | --- |
| MOV AX, [0200] | A10002 |
| MOV BX, [0202] | 8B1E0202 |
| ADD BX,AX | 01C3 |
| MOV [0204], BX | 891E0402 |
| NOP | 90 |

Table (6)

- Activity 2.4:

The 8-bit data value stored at DS: 0200 after the data in Activity 2.1 has been entered is 1B H. Since the 8086 MP is little endian; the low byte is stored at low address.

- Activity 2.5:

The16-bit data value stored at DS: 0200 after the data in Activity 2.1 has been entered is 1B 9F H. Since; [200] = 1B, and [201]= 9F. Low byte is at low address (Little Endian).

- Activity 2.6:

The 8-bit data value stored at DS: 0204 after the data in Activity 2.1 has been entered is 51 h.  Since; [200] = 1B H, and [201] = 9F H. [202] = 36 H, and [203] = 4A H. Low byte is at low address (Little Endian).

- MOV AX, [0200].  AX=9F1B H.

- MOV BX, [0202]. BX=4A36 H.

- ADD BX, AX.      BX=9F1B H +4A36 H= E951.

- MOV [0204], BX. [204] = 51. (1 Byte data).

- Activity 2.7:

The content of register AX after the instruction A10002 is executed:

MOV AX, [0200].  AX = 9F1B H.

- Activity 2.8:

Before we do these instructions: [204] = 00 H.

The content of memory offset 0204 after each instruction is as shown in the table below (Table (7)):

|  | A10002 | 8B1E0202 | 01C3 | 891E0402 |
| --- | --- | --- | --- | --- |
| DS: 204 | 00 | 00 | 00 | 51 |

Table (7)

Only the last instruction affects the value stored in address 204.

mov [204], bx.

- Part 3: Entering assembly code in DEBUG.

- Activity 3.1:

We enter the following assembly code using DEBUG command A at offset 100H:

MOV CL, 42

MOV DL, 2A

ADD CL, DL

NOP

- Activity 3.2:

We execute the program using the T command. And determine the content of CL, DL, and IP. The results are as shown in the table below:

|  | MOV CL,42 | MOV DL,2A | ADD CL,DL |
| --- | --- | --- | --- |
| CL | 42 | 42 | 6C |
| DL | 00 | 2A | 2A |
| IP | 102 | 104 | 106 |

Table (8)

The initial values of CL, DL, and IP are 00H, 00H, and 100H respectively.

After executing MOV CL, 42 CL= 42H, IP becomes 102H the size of this instruction is 2 bytes.

After executing MOV DL, 2A CL= 42H, DL=2AH, and IP becomes 104H the size of this instruction is 2 bytes.

After executing ADD CL, DL CL= 6CH, DL=2AH, IP becomes 102H the size of this instruction is 2 bytes. CL= 42H + 2AH= 6CH.

- Conclusion:

At the end of this experiment, we were more familiarized with the DEBUG program and introduced to its different command which each one has a different use. DEBUG program is very useful in trapping the code and see the effect of each simple instruction.