بسم الله الرحمن الرحيم

Computer System Engineering Department

ENCS 211

DIGITAL ELECTRONICS AND COMPUTER ORGANIZATION LAB

Report For Experiment NO.12

Arithmetic Elements.

- Students name & ID: Hanadi Mzahim, 1071152 & Eman Arar, 1070720.

- Instructor Name:  MR. Mohammad Abu-Ayyash.

- Teacher Assistant: ENG. Anas Abd-elrazeq.

- Date: 5-5-2010.

- Section: 2.

- Procedure:

Arithmetic Logic Unit (ALU) Circuit:

Arithmetic logic unit (ALU) is a digital circuit that performs arithmetic and logical operations. In this experiment, the 74181 ALU IC will be used to introduce the concept of ALU. Its logic diagram is shown in Fig (1).

Fig (1)

The KL-26005 Module was set on the KL-22001 Basic Electricity Circuit Lab, and block b was located. Function-select lines S3-S0 were connected to data switches SW7-SW4, respectively. M was connected to SW3 for selecting arithmetic or logic operation. When M = “0” arithmetic operation is performed. When M = “1” logic operation is performed.

Inputs A3-A0 and B3-B0 were connected to D7-D4 and D3-D0, respectively. Cn was connected to “0”. Outputs F3-F0 were connected to logic Indicators L4-L1 and Cn+4 to L8. (Inputs A3-A0, B3-B0, and outputs F3-F0 are active low).

M was set to “1” to perform the following function:

- When S3S2S1S0= 0000, A3A2A1A0=0000 and B3B2B1B0=1111, the outputs F3F2F1F0=            1111         .

- When S3S2S1S0= 0000, A3A2A1A0=1100 and B3B2B1B0=1010, the outputs F3F2F1F0=              0011       .

- When S3S2S1S0=1001, A3A2A1A0=1100 and B3B2B1B0=0110, the outputs F3F2F1 F0=               0101           .

What is the relationship between the inputs and outputs in terms of logic?

F = (AB)’       .

- When S3S2S1S0=1011, A3A2A1A0=0011 and B3B2B1B0=1001, the outputs F3F2F1F0 =        0001    .

What is the relationship between the inputs and outputs in terms of Logic?

F = AB          .

M and Cn were set to “0” and the previous carry was ignored. Then, the following arithmetic functions were performed.

- When S3S2S1S0=1001, A3A2A1A0=B3B2B1B0=0100, the outputs

F3F2F1 F0 =         1001       and Cn+4 =          1         .

- When S3S2S1S0=1001, A3A2A1A0=1000 and B3B2B1B0=1100, the

Outputs F3F2F1F0 =         0101      and Cn+4 =            0         .

- When S3S2S1S0=0011, A3A2A1A0=0100 and B3B2B1B0=0010, the

Outputs F3F2F1 F0 =      0000      and Cn+4 =        0        .

- When S3S2S1S0=0011, A3A2A1A0=1010 and B3B2B1B0=1000, the

Outputs F3F2F1 F0 =          0000       and Cn+4 =        0        .

- When S3S2S1S0=0000, A3A2A1A0=1010 and B3B2B1B0=0011 , the

Outputs F3F2F1 F0 =        1011      and Cn+4=        1          .

M was set to “0” and Cn was set to “1”. The inputs sequences in table (1) were followed and outputs were recorded.

Table (1)

Depending on the state of M and Cn, function-select lines S0-S3 have different functions. The function table of 74181 is shown in table (2).

Table (2)

Parity Generator Circuit:

A parity bit is a bit that is added to ensure that the number of bits with the value one in a set of bits is even or odd. Parity bits are used as the simplest form of error detecting code.

There are two variants of parity bits: even parity bit and odd parity bit. When using even parity, the parity bit is set to 1 if the number of ones in a given set of bits (not including the parity bit) is odd, making the entire set of bits (including the parity bit) even. When using odd parity, the parity bit is set to 1 if the number of ones in a given set of bits (not including the parity bit) is even, making the entire set of bits (including the parity bit) odd.

A parity bit is generated by the parity generator. There are two types of parity generators. The "Odd" parity generator will generate a "1" if the data contains an even number of "1"s. For example, data "10111011" has six "1"’s. When the parity bit is added to the end of this data, the number of "1"s in the data will become an "Odd" number, hence the name "Odd Parity Generator".

On the other hand, an Even parity generator will add a "1" to data with odd number of "1"s to make the total number of "1"s even. If the data already has an even number of "1"s no parity bit is generated. Output Y of the Even parity generator shown in Fig (2) will be 0 if the inputs ABCDEFGH is equal to

10111011.

Fig (2)

- Parity Generator Constructed with XOR Gates:

The KL-26002 was set on the KL-22001 Basic Electricity Circuit Lab, and block a was located. The connection was completed by referring to the wiring diagram in Fig (3) and the even parity generator circuit was shown in Fig (4). +5VDC was applied from the fixed power on KL-22001 lab to KL-26002 Module.

Fig (3)

Fig (4)

Inputs A, B, C, D and E were connected to data switches SW0-SW4 and output F6 was connected to logic indicator L1. The input sequences in table (3) were followed and the outputs were recorded.

Table (3)

- Parity Generator IC:

The KL-26005 Module was set on the KL-22001 Basic Electricity Circuit Lab, and block c was located.U7 is a parity generator IC.

Input A1, B1, C1, D1, E1, F1, G1, H1 and L1 were connected to data switches SW0-SW7 and D7, respectively. Output Y0 was connected to L1 and Y1 was connected L2. +5VDC was applied from the fixed power on KL-22001 lab to KL-26002 Module.

The input sequences in table (4) were followed and the outputs were recorded.

Table (4)

- Conclusion:

At the end of this experiment, we understood the functions and applications of the ALU (Arithmetic Logic Unit). We performed arithmetic and logic operations using the 74181 ALU IC. We understood the construction and the application of parity generators. We generated parity bit using XOR gates and parity generator IC.