بسم الله الرحمن الرحيم

Computer System Engineering Department

ENCS 211

DIGITAL ELECTRONICS AND COMPUTER ORGANIZATION LAB

Report For Experiment NO.2

Comparators, Adders & Subtractors.

- Students name & ID: Hanadi Mzahim, 1071152 & Eman Arar, 1070720.

- Instructor Name:  MR. Mohammad Abu-Ayyash.

- Date: 3-2-2010.

- Section: 2.

- Procedure:

- Constructing Comparator with Basic Logic Gates.

As a first step, we Set the KL-26001 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block a , then we  completed the connections by referring to wiring diagram in Fig 1.5(a).

- Wiring diagram (KL-26001 block a)

(b) Logic Diagram

FIG 1.5: 1-bit comparator

The inputs were active high.  We Connected inputs A and B to Data Switches SW1 and SW2 (The outputs are active low). Then we Connected outputs F1, F2, F5 to logic Indicators L1, L2, L3, respectively and we Applied +5 VDC from the Fixed Power on KL-26001 Module. We followed, the input sequences in Table 1.1. And we recorded the outputs we observed.

| INPUTS B A SW2 SW1 |  | OUTPUTS F1 F2 F5 (L1) (L2) (L3) |
| --- | --- | --- |
| 0 0 | A=B | 1 1 0 |
| 0 1 | A>B | 0 1 1 |
| 1 0 | A<B | 1 0 1 |
| 1 1 | A=B | 1 1 0 |

Table 1.1

We note from the last table that f5 is active when A =B, at AB + A'B' = A XOR B

F1 is active at AB’, and F2 is active at A’B, and fig 1.5 (b) represents the logic diagram of those functions.

(B) Constructing Comparator with TTL IC.

Then, we Set the KL26005 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block a), we apply +5VDC from the Fixed Power on KL-22001 Lab to KL26005 Module. U6 is a 7485 4-bit comparator IC. Its pin assignment and function table are given below:

FIG1.6: 26005 block a

We connected the inputs A1~A4 to SW4 ~ SW7 and B1 ~ B4 to SW0 ~ SW3, Respectively. And we Connected the outputs A=B to L1, A<B to L2, and A>B to L3. We followed the input sequences in Table1.2. We observe and recorded the results :

| Inputs A4 A3 A2 A1 B4 B3 B2 B1 SW7 SW6 SW5 SW4 SW3 SW2 SW1 SW0 | OUTPUTS L3 L2 L1 A>B A<B A= B |
| --- | --- |
| 0 0 0 0 0 0 0 0 | 0 0 1 |
| 0 1 0 1 0 0 1 1 | 1 0 0 |
| 0 1 0 0 1 0 0 0 | 0 1 0 |
| 1 0 1 0 1 0 1 0 | 0 0 1 |
| 0 1 1 0 1 0 1 1 | 0 1 0 |
| 1 1 1 1 1 1 0 0 | 1 0 0 |

Table 1.2

1.5.2. Half- and Full-Adder Circuits

A. Constructing Half- and Full-Adders with Basic logic Gates

We  Set the KL-26002 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block a. we Completed  the connections by referring to the wiring diagram in Fig1.7, we Applied +5VDC from Fixed Power on the KL-22001 Lab to KL-26002 Module.

FIG1.7: Wiring Diagram (KL-26002 Block a)

FIG1.8: Half-Adder Circuit

We connected inputs A and B to Data Switches SW0 and SW1, respectively.

And we Connected output F1 and F2 to logic Indicators L1 and L2.,we Followed the input sequence for A and B in Table1.3 and recorded  the outputs:

| Inputs SW1(B) SW0(A) | OUTPUTS CARRY(F1) SUM(F2) |
| --- | --- |
| 0 0 | 0 0 |
| 0 1 | 0 1 |
| 1 0 | 0 1 |
| 1 1 | 1 0 |

Table 1.3

From the previous table  we can note that the summation of two numbers of one bit  , is representing by  XOR gate , SUM = A XOR B. the carry = A AND B  , and that is shown in fig 1.8.We completed the connections by referring to the wiring diagram in Fig1.10 and the full-adder circuit in Fig1.9.

FIG1.9: Full-Adder Circuit

FIG1.10: Wiring Diagram (KL-26002 Block a)

We Connected A, B & C to SW1, SW2, and SW3. The input A represents the augend, input B the addend, and the C is the previous carry. We connected the outputs F3 and F5 to Logic Indicators L1 and L2, respectively.

We followed the input sequence in Table1.4 and recorded the output states:

| INPUT SW3 © SW2(B) SW1(A) | OUTPUT CARRY (F3) SUM(F5) |
| --- | --- |
| 0 0 0 | 0 0 |
| 0 0 1 | 0 1 |
| 0 1 0 | 0 1 |
| 0 1 1 | 1 0 |
| 1 0 0 | 0 1 |
| 1 0 1 | 1 0 |
| 1 1 0 | 1 0 |
| 1 1 1 | 1 1 |

Table 1.4

After using the k_map we get the function for the carry and sum we get:

Carry = AC + BC +AB.

SUM = CA'B' + C'AB' + CBA +C'BA' = C'(B'A + BA') + C (A'B' +AB)

= C XOR B XOR A.

And the diagram in fig(1.9) shows those functions .

- Constructing 4-Bit Full-Adder with IC

At this part we Set the KL-26002 Module on the KL-22001 Basic Electricity Lab, and locate block b. The U5, 7483 is a 4-bit binary adder. We connected input Y5 to ground “0”, so the XOR gates of U6, which are connected to Y0~Y3, will act as buffers. Then, we Connected inputs X0~X3 (addend) and Y0~Y3 (augends) to Data Switches SW0~SW3 and SW4~SW7 respectively. We connected F1(Carry out) to L1 and Σ0 ~ Σ3(sum) to L2~L5. Apply +5VDC from the Fixed Power on KL-22001 Lab to KL-26002 Module. We followed input sequences in Table 1.6 and record the outputs F1 in binary and Σ in hexadecimal.

X=X3X2X1X0, Y=Y3Y2Y1Y0, Σ= Σ3Σ2Σ1Σ0.

FIG1.12: Wiring Diagram (KL-26002 block b)

| INPUT X Y | OUTPU F |
| --- | --- |
| 0000 0000 0000 0001 0000 0110 0000 1001 0000 1111 0001 0011 0001 0110 0001 1000 0011 0110 0100 1000 0100 1111 1000 0111 1001 1001 1010 1011 1100 1110 1111 1111 | 0000 0 0001 0 0110 0 1001 0 1111 0 0100 0 0111 0 1001 0 1001 0 1100 0 0011 1 1111 0 0010 1 0101 1 1010 1 1110 1 |

Table 1.6

- Constructing BCD Adder

We set the KL-26002 Module on the KL-22001 Basic Electricity Circuit

Lab, and locate block b. The circuit, shown in Fig1.12, will act as

a BCD adder. We Connected inputs X0~X3 to SW0~SW3, Y0~Y3 to SW4~SW7, Y5 to ground (“0”).U5 and U9 are 7483 4-bit binary full adder, we connected outputsF8~F11 of U5 to the inputs of one of the digital displays. F8~F11 are also connected to Logic Indicators L1~L4. We Connected F1and F2 to logic Indicators L5 and L6, respectively. We also connected outputs F4~F7 of U9 to inputs of another Digital Display.

Then, we connected F4~F7 to L0~L3 and F3 to L4. F11~F8 are the sum of X0~X3 added to Y0~Y3 while F1 is the carry. We followed the input sequences for X0~X3 and Y0~Y3 in the table 1.7, and record the output state.

| X3 X2 X1 X0 | Y3 Y2 Y1 Y0 | OUTPUTS (U5) F1 F11 F10 F9 F8 | FINAL F2 F3 F7 F6 F5 F4 |
| --- | --- | --- | --- |
| 0 0 0 0 | 0 0 0 0 | 0 0 0 0 0 | 0 0 0 0 0 0 |
| 0 0 0 1 | 0 0 1 1 | 0 0 1 0 0 | 0 0 0 1 0 0 |
| 0 0 1 1 | 0 1 0 0 | 0 0 1 1 1 | 0 0 0 1 1 1 |
| 0 0 1 0 | 0 0 1 0 | 0 0 1 0 0 | 0 0 0 1 0 0 |
| 0 0 1 0 | 1 0 0 0 | 0 1 0 1 0 | 1 1 0 0 0 0 |
| 0 0 1 1 | 0 1 1 0 | 0 1 0 0 1 | 0 0 1 0 0 1 |
| 0 1 0 0 | 0 0 1 0 | 0 0 1 1 0 | 0 0 0 1 1 0 |
| 0 1 0 0 | 0 1 0 1 | 0 1 0 0 1 | 0 0 1 0 0 1 |
| 0 1 0 0 | 0 1 1 0 | 0 1 0 1 0 | 0 1 0 0 0 0 |
| 0 1 0 1 | 0 1 1 0 | 0 1 0 1 1 | 0 1 0 0 0 1 |
| 0 1 1 0 | 0 1 1 1 | 0 1 1 0 1 | 0 1 0 0 1 1 |
| 0 1 1 1 | 1 0 0 0 | 0 1 1 1 1 | 0 1 1 0 0 0 |
| 0 1 1 1 | 1 0 0 1 | 1 0 0 0 0 | 1 1 0 1 0 1 |
| 1 0 0 0 | 1 0 0 1 | 1 0 0 0 1 | 1 1 0 1 1 1 |
| 1 0 0 1 | 1 0 0 1 | 1 0 0 1 0 | 1 1 1 0 0 0 |
| 1 0 1 0 | 1 0 1 0 | 1 0 1 0 0 | 1 1 1 0 1 0 |
| 1 0 1 0 | 1 0 1 1 | 1 0 1 0 1 | 1 1 1 0 1 1 |
| 1 0 1 0 | 1 1 0 0 | 1 0 1 1 0 | 1 1 1 1 0 0 |
| 1 0 1 1 | 1 1 1 0 | 1 1 0 0 1 | 1 1 1 1 1 1 |
| 1 1 1 1 | 1 1 1 1 | 1 1 1 1 0 | 1 0 0 1 0 0 |

Table (1.7)

After that, we set the KL-26002 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block a. And complete the connections by referring to the wiring diagram in Fig1.13. Then, apply +5VDC from Fixed Power on KL-22001 Lab to KL-26002 Module.

FIG1.13: Wiring Diagram (KL-26002 block a).

After that, we connect inputs A~C to Data Switches SW0~SW2; outputs F2 to L1, F1 to L2, F3 to L3 & F5 to L4. And follow the input sequences as shown in Table.(1.9).

| Inputs |  |  | BW1 | Dif1 | BW2 | Dif2 |
| --- | --- | --- | --- | --- | --- | --- |
| C | A | B | F1 | F2 | F3 | F5 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 1 | 1 |

Table.(1.9)

We note that, when C=0 the circuit is half-Subtractor with the borrow output F1 (BW1), and the differences output F2 (DF1) =A-B. And when C=1 the circuit is a full-Subtractor with borrow output F3 (BW2) and the differences output F5 (DF2) = A-B-C.

After that, we set the KL-26002 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block b. And complete the connections by referring to the wiring diagram in Fig1.14. Then apply +5VDC from the Fixed Power on KL-22001 lab to KL-26002 Module.

FIG1.14: Wiring Diagram (KL-26002 block b).

Then, we connect inputs X3~X0 to SW7~SW4; Y3~Y0 to SW3~SW0. And connect outputs F1 to L1; F11 to L5~L2. To execute the subtract operation, we connect Y5 to +5V (“1”) (or Cin of U5=1). And follow the input sequences and record the output states in Table 1.10.

| Inputs |  |  |  |  |  |  |  | Borrow | Difference |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X3 | X2 | X1 | X0 | Y3 | Y2 | Y1 | Y0 | F1 | F11 | F10 | F9 | F8 |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |

Table.(1.10)

- A 4-input, 3-output circuit that compares 2-bit unsigned numbers and output (1) on one of three output lines according to whether the first number is greater than, equal to, or less than the other number. You can only use two 4×1 multiplexers.

| A |  | B |  | A>B | A<B |
| --- | --- | --- | --- | --- | --- |
| A1 | A0 | B1 | B0 | F1 | F2 |
| 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 |

Table.(1.11)

Take A1, A0 are the selection lines for the two muxes.

When A1A0= 00: (D0)

| 0 | 0 |
| --- | --- |
| 0 | 0 |

F1 = 0.

When A1A0= 01: (D1)

| 1 | 1 |
| --- | --- |
| 1 | 0 |

F2 = B1'+B0'.

| 1 | 0 |
| --- | --- |
| 0 | 0 |

| 0 | 1 |
| --- | --- |
| 1 | 0 |

F1= B1'B0'.

When A1A0= 10: (D2)

| 1 | 1 |
| --- | --- |
| 0 | 0 |

F1= B1'.

F2=B1'B0 + B1B0'.

= B1⊕B0.

| 0 | 0 |
| --- | --- |
| 0 | 1 |

F2=B1B0.

When A1A0= 11: (D3)

| 1 | 1 |
| --- | --- |
| 1 | 0 |

F1= B1'+ B0'.              F2= 0.

Fig.(1.15)

- Conclusion:

In this experiment, we note that we use basic gates (and, or, xor & nand) to implement the circuit of full-adder, half-adder, half-Subtractor & full-Subtractor. Also, we use IC‘s to construct these circuits (full-adder, half-adder, half-Subtractor, full-Subtractor, 4-bits full-adder, 4-bits Subtractor & BCD adder).

We do not face any problem with basic gates, but when we use IC exactly BCD adder, we had; since there are many output to record. We hope that the results are true.