1-2  NAND Gate circuit

After that, we set the kl _26001 module on the kl_22001 basic electricity circuit lab ,and locate block b we inserted the bridging plug shown in fig 1_2_2(a) , we applied +5VDC from the fixed power on kl_22001 lab to kl _26001 module .

(a) Wiring diagram (KL-26001 block b)

(b) NOT gate constructed with NAND gate

Fig. 1-2-2 NOT gate constructed with NAND gate

Then we Connected input A to Data switch SW1 and output F2 to Logic Indicator

L1.  We Observed  and recorded  the output states.

When SW1 ="0", F2 = ______1_______

When SW1 ="1", F2 = _______0_______

From the previous results we not that when we connect both inputs of the nand gate as one input ,it works as a not gate ,since (AB)' at A=b , is equivalent to (A A)' = A' , and that is shown at the gate on the left of fig (b) above.

Then We removed the bridging plug between A and A1,and connected input A1 to +5v (“1”) , the other connections were unchanged ,we observed and recorded the output states:

When SW1 ="0", F2 = ______1_______

When SW1 ="1", F2 = _______0_______

The previous  results shows that when we let one of the inputs of the nand gate to be “1” ,nand gate works as a not gate , since (A. 1)' = A', the figure that represent that is in the right of fig 1-2-2(b).

We completed the connections as shown in Fig. 1-2-3(a) and Connected A  to SW1, A1 to SW2 and F4 to L1.

- Wiring diagram (KL-26001 block b)

- Equivalent to an AND gate

Fig. 1-2-3 AND gate constructed with NAND gates

We Followed  the input sequences and recorded the outputs in Table 1-2-1.

| SW2(A1) | SW1(A) | F4 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

1-2-1

we note from the results in the above truth table that a combination of two nand gates let it to work as an and gate , since (1) (AB)' = A' + B' , (2) ((A' + B'). (A' + B') ' = (A' + B')' +(A' + B')' = (AB) + (AB) = (AB) .(according to demorgans’ low) ,the diagram of fig 1-2-3 (b) shows that .

We Completed the connections by referring to the wiring diagram in Fig. 1-2-4(a) and

the circuit in Fig. 1-2-4(b) and  Connected. A to SW1; D to SW2, and F4 to L1.

(a) Wring diagram (KL-26001 block b)

(b)Equivalent to an OR gate

Fig 1-2-4 OR gate constructed with NAND gates

We Followed  the input sequences and recorded the outputs in Table 1-2-2

| SW2(A1) | SW1(A) | F4 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

1-2-2

we note from the results in the above truth table that a combination of three nand gates let it to work as an OR gate , since (1) (AA)'  = A' , (2) (BB)' = B', (3) (A'B') '  = A+B  (demorgans law).(according to demorgans’ low) ,the diagram of fig 1-2-4 (b) shows that.

1-3 XOR Gate circuit

A. Set the KL-26001 Module on the KL-22001 Basic Electricity Circuit Lab, and

Locate block b. Complete the connections by referring to the wiring diagram in

Fig. 1-3-3(a). Connect inputs A to SW1, D to SW2; outputs F1 to L1, F2 to L2; F3 to L3 and F4 to L4. Apply +5VDC from the Fixed Power to KL-26001 Module.

(a) Wiring diagram (KL-26001 block b)

(b) Equivalent circuit

Fig.1-3-3 XOR gate constructed with NAND gates

After following the input of sequences for A and 0 in table 1-3-1 and record the outputs.

| SW2(D) | SW1(A) | F1 F2 F3 F4 |
| --- | --- | --- |
| 0 | 0 | 1 1 1 0 |
| 0 | 1 | 1 1 0 1 |
| 1 | 0 | 1 0 1 1 |
| 1 | 1 | 0 1 1 0 |

Table 1-3-1

The truth table 1-3-1 sows that XOR gate can be implemented by three nand gates ,if we use fig 1-3-3 (b) to show that : (AD) ' = A' + D' = F1 , (A.F1)' = (A . (A'+ D'))' = A' + AD = F2.

(D . (A' + D'))' = D' + AD = F3 .

((A' + AD)(D' + AD))' = ((AD) '. D)+ (A. (AD)')

= (A' + D') .D +A. (A' + D') = A'D + D'D  + AA' +AD' = A'D + AD' which represents xor gate .

Then we set the KL-26001 Module on the KL-22001 Basic Electricity Circuit Lab, and

locate block a. Complete the connections by referring to the wiring diagram in

Fig. 1-3-4(a) Apply +5VDC from the Fixed Power on KL-22001 Lab to KL-26001 Module We connected inputs A, B to SW1, SW2; outputs F1. F2. F3 to L1. L2. L3.

(a) Wiring diagram (KL-26001 block a)

.

- Equivalent circuit

Fig.1-3-4 XOR gale constructed with basic gates

We Followed  the input sequences for A and B in Table 1-3~2 and record the outputs.

| SW2(B) | SW1(A) | F1 F2 F3 |
| --- | --- | --- |
| 0 | 0 | 1 1 0 |
| 0 | 1 | 0 1 1 |
| 1 | 0 | 1 1 0 |
| 1 | 1 | 0 1 1 |

TABLE 1-3-2

from the previous truth table we note that we can get xor gate by the basic gates ,since in fig 1-3-4(B) , (A'B) = F1 , (AB') =F2 , F1 + F2 = A'B + AB' = Xor (A,B)