بسم الله الرحمن الرحيم

Computer System Engineering Department

ENCS 211

Digital Lab

Report for Experiment NO.1

Combinational Logic Circuits.

- Students name & ID: Hanadi Mzahim, 1071152 & Eman Arar, 1070720.

- Instructor Name:  MR. Mohammad Abu-Ayyash.

- Date: 27-1-2010.

- Section: 2.

- Procedure:

- Part 1: NOR GATE.

We set the KL-26001 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block c. Then, we apply +5VDC from the Fixed Power on KL-22001 Lab toKL-26001 Module. And connect inputs A3, A4 on U2 gate to Data Switches SW0, SW1 and the output F2 to Logic Indicator L1. And set SW0 to ''0'' and observe states of F2 at SW1 ="0" and SW1 ="1", logically as shown in fig.1b.

- KL-26001 block c

Fig. (1)

The Results are:

- When SW1="0", F2= _______1________.

- When SW1="1", F2= _______0________.

We conclude from our observation, that NOR gate can be used as NOT gate, if we connect the circuit of fig.21; since (A3+A4) ' =A3', when A4=0.

Then, we complete the connections by referring to the wiring diagram in Fig.2 (a) and the circuit in Fig.2 (b). This is done by connecting A3 and A4 together (A3=A4), and connect A3 to Data Switch SWO and the output F2 to Logic Indicator L1.

- Wiring diagram (KL-26001 block c)

(b)

Fig. (2)

The results obtained are:

- When SW0="0", F2=______1_______.

- When SW0="1", F2=_______0______.

We conclude from our observation, that NOR gate can be used as NOT gate, if we connect the circuit of fig.2b; since (A3+A4) ' = (A3+A3)'=A3', when A4=A3.

After that, we complete the connections by referring to the wiring diagram in Fig.3 (a) and the circuit in Fig.3 (b). Connect A3 to Data Switch SW0 and the output F4 to Logic Indicator L1.

- Wiring diagram(KL-26001 block c)

Fig. (3)

- When SW0= "0", F4 =_______0______.

- When SW0= "1", F4 =_______1______.

We conclude from our observation, that NOR gate can be used as Buffer, if we connect the circuit of fig.3b; since

((A3+A4) ' + (A3+A4) ') '= ((A3+A3) ' + (A3+A3) ') '= ((A3+A3) ')'= A3, when A4=A3.

Then, we complete the connections by referring to the wiring diagram in Fig.4 (a) and the circuit in Fig.4 (b). And connect inputs A3 to SW0, A4 to SW1; and output F4 to Logic Indicator L1, then, follow the sequences of input A3 & A4 as in table.1, then complete it.

- Wiring diagram (KL-26001 block c)

Fig. (4)

| SWO(A3) | SW1 (A4) | F4 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Table. (1)

- Part 2: NAND GATE Circuits.

We set the kl26001 module on the kl_22001 basic electricity circuit lab ,and locate block b we inserted the bridging plug shown in fig.(5), we applied +5VDC from the fixed power on kl_22001 lab to kl 26001 module .

(a) Wiring diagram (KL-26001 block b)

(b)

Fig. (5)

Then we Connected input A to Data switch SW1 and output F2 to Logic Indicator

L1.  We Observed and recorded the output states.

When SW1 ="0", F2 = ______1_______.

When SW1 ="1", F2 = _______0______.

From the previous results we not that when we connect both inputs of the nand gate as one input, it works as a not gate, since (AB)' at A=b, is equivalent to (A A)' = A’, and that is shown at the gate on the left of fig (b) above.

Then we removed the bridging plug between A and A1, and connected input A1 to +5v (“1”), the other connections were unchanged, we observed and recorded the output states:

When SW1 ="0", F2 = ______1_______.

When SW1 ="1", F2 = _______0______.

The previous  results shows that when we let one of the inputs of the nand gate to be “1” , nand gate works as a not gate , since (A. 1)' = A', the figure that represent that is in the right of fig.5(b).

We completed the connections as shown in Fig. (6) And Connected A to SW1, A1 to SW2 and F4 to L1.

- Wiring diagram (KL-26001 block b)

Fig.(6)

We Followed the input sequences and recorded the outputs in Table. (2).

| SW2(A1) | SW1(A) | F4 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

Table. (2)

We note from the results in the above truth table that a combination of two nand gates let it to work as an and gate, since (1) (AB)' = A' + B',

(2) ((A' + B'). (A' + B')) '= (A' + B')' + (A' + B')' = (AB) + (AB) = (AB). (According to De Morgan’s low), the diagram of fig.6 (b) shows that.

We Completed the connections by referring to the wiring diagram in Fig.7 (a) and

The circuit in Fig.7 (b) and Connect A to SW1; D to SW2, and F4 to L1.

(a) Wring diagram (KL-26001 block b)

(b)

Fig. (7)

We Followed the input sequences and recorded the outputs in Table.(3)

| SW2(A1) | SW1(A) | F4 |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

Table.(3)

We note from the results in the above truth table that a combination of three nand gates let it to work as an OR gate, since (1) (AA)' = A’, (2) (BB)' = B', (3) (A'B') ‘= A+B (De Morgan’s law), the diagram of Fig.7(b) shows that.

- Part 3: XOR GATE Circuits.

We set the KL-26001 Module on the KL-22001 Basic Electricity Circuit Lab, and

Locate block b. Complete the connections by referring to the wiring diagram in

Fig.(8). Connect inputs A to SW1, D to SW2; outputs F1 to L1, F2 to L2; F3 to L3 and F4 to L4. Apply +5VDC from the Fixed Power to KL-26001 Module.

(a) Wiring diagram (KL-26001 block b)

(b) Equivalent circuit

Fig.(8)

After following the input of sequences for A and D in Table.(4) and record the outputs.

| SW2(D) | SW1(A) | F1 F2 F3 F4 |
| --- | --- | --- |
| 0 | 0 | 1 1 1 0 |
| 0 | 1 | 1 1 0 1 |
| 1 | 0 | 1 0 1 1 |
| 1 | 1 | 0 1 1 0 |

Table.(4)

The truth table.4 sows that XOR gate can be implemented by three nand gates, if we use fig.8(b) to show that :

(AD) ' = A' + D' = F1, (A.F1)' = (A. (A'+ D'))' = A' + AD = F2.

(D. (A' + D'))' = D' + AD = F3.

((A' + AD) (D' + AD))' = ((AD) '. D) + (A. (AD)')

= (A' + D') .D +A. (A' + D') = A'D + D'D + AA' +AD'

= A'D + AD' (which represents xor gate).

Then we set the KL-26001 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block a. Complete the connections by referring to the wiring diagram in Fig.9(a). Apply +5VDC from the Fixed Power on KL-22001 Lab to KL-26001 Module and we connected inputs A, B to SW1, SW2; outputs F1, F2 & F3 to L1, L2 & L3.

(a) Wiring diagram (KL-26001 block a)

.

(b) Equivalent circuit

Fig.(9)

We Followed the input sequences for A and B in Table.(5), and record the outputs.

| SW2(B) | SW1(A) | F1 F2 F3 |
| --- | --- | --- |
| 0 | 0 | 1 1 0 |
| 0 | 1 | 0 1 1 |
| 1 | 0 | 1 1 0 |
| 1 | 1 | 0 1 1 |

Table.(5)

From the previous truth table we note that we can get xor gate by the basic gates; since in fig.9 (b), (A'B) = F1, (AB') =F2, F1 + F2 = A'B + AB' = Xor (A, B)

- Part 4: AOI GATE Circuits.

First, we set the KL-26001 Module on the KL-22001 Basic Electricity Circuit

Lab, and locate block a as shown in fig.10 (a) & (b). Then, we connect inputs A,

A1, B, B1 to Data Switches SW0, SW1, SW2, SW3 respectively, and outputs

F3, F4 to Logic Indicators L1, and L2. And apply +5VDC from the Fixed Power

On KL-22001 Lab to KL-26001 Module. Then set B.B1 to "0", follow the input

Sequences for A and A1 in Table. (6) And record the outputs.

(a) Wiring diagram (KL-26001 block a)

(b) Equivalent circuit

(c) Actual circuit

Fig.(10)

| A1 | A | F3 | F4 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

Table.(6)

We note that F3 works as AND gate between A1 & A when B.B1=0; since

F3= ((AA1)’. (BB1)')' = ((AA1)’. 1)'= AA1. Also, F4=F3'.

When BB1≠0 F3≠ AA1; since F3= ((AA1)’. 0)'= (0)'= 1.

When A-A1=0, follow the input sequences for B and B1 in Table. (7), and

Record the outputs.

| B1 | B | F3 | F4 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 |

Table.(7)

We note that F3 works as AND gate between B1 & B when A.A1=0; since

F3= ((AA1)’. (BB1)')' = ((BB1)’. 1)'= BB1. Also, F4=F3'.

When AA1≠0 F3≠ BB1; since F3= ((BB1)’. 0)'= (0)'= 1.

F3= ((AA1)’. (BB1)')'= ((AA1)')' + ((BB1)')'= AA1+BB1.

- Part 5: LOGIC Problem & Design.

A given logic system is shown in Fig. 1-5-1; output of the system F is in”1”state in each of the following condition:

Fig. (1-5-1)

- C and D are in the “1” state.

- A, B and D are in the “1” state, C is in "0" state.

- B and D are in the "1" state, A and C are in "0" state.

- C and B are in the “1” state, A and D are in “0” state.

- C is in the “1” state, A, B and D are in the “0” state.

- Complete the truth table of the above logic system.

Table.(8) shows the truth table of the above function.

| A | B | C | D | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 (m0) |
| 0 | 0 | 0 | 1 | 0 (m1) |
| 0 | 0 | 1 | 0 | 1 (m2) |
| 0 | 0 | 1 | 1 | 1 (m3) |
| 0 | 1 | 0 | 0 | 0 (m4) |
| 0 | 1 | 0 | 1 | 1 (m5) |
| 0 | 1 | 1 | 0 | 1 (m6) |
| 0 | 1 | 1 | 1 | 1 (m7) |
| 1 | 0 | 0 | 0 | 0 (m8) |
| 1 | 0 | 0 | 1 | 0 (m9) |
| 1 | 0 | 1 | 0 | 0 (m10) |
| 1 | 0 | 1 | 1 | 1 (m11) |
| 1 | 1 | 0 | 0 | 0 (m12) |
| 1 | 1 | 0 | 1 | 1 (m13) |
| 1 | 1 | 1 | 0 | 0 (m14) |
| 1 | 1 | 1 | 1 | 1 (m15) |

Table. (8)

- Write the Boolean equation as canonical sum.

F (A, B, C, D) = ∑ (2, 3, 5, 6, 7, 11, 13, 15)

=A'B'CD'+ A'B'CD+ A'BC'D+ A'BCD'+ A'BCD+ AB'CD+

ABC'D+ ABCD.

- Minimize the expression that you obtained using laws of Boolean algebra. Write down reduced equation.

F= A'B'CD'+ A'B'CD+ A'BC'D+ A'BCD'+ A'BCD+ AB'CD+

ABC'D+ ABCD

= A'B'C (D+ D') + A'BC (D+D') + BC'D (A+A') + ACD (B+B')

= A'C + ACD + BC'D

- Draw the k-map of logic problem and indicate columns and rows circle the sub-cubes and write down the equation that you obtained is the equation identical to the one that you obtained in paragraph 3.

| 0 | 0 | 1 | 1 |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 0 |

K-map.

F (A, B, C, D) = A'C+ CD+ BD.

The two functions in 2 & 3 are not the same, but they give the same outputs for F.

- Implements the reduced expression that you obtained in the previous paragraph using NAND gates only.

Fig.(11)

Using NAND gates:

Fig. (12)

After finding Boolean expression for F, we design it using NAND gates as shown in fig. (12), and check the output at Different sequence of inputs with.

We find that our Design is correct, and all sequences of input give a correct output.

- Conclusion:

- AND:

AND gate can be constructed using two types of Transistors: NMOS or PMOS MOSFETS.

- OR:

CMOS OR Gate

OR gate can be constructed using three types of Transistors: NMOS or PMOS or CMOS FETS.

- INVERTER:

INVERTER gate can be constructed using four types of Transistors:

- One NMOSFET Transistor connected with a series resistor.

- One PMOSFET Transistor connected with a series resistor.

But these two types 1 & 2 has two drawback:

- Drain (series) resistor consumes the power.

- Drain (series) resistor also affects the processing speed badly.

- Two complementary CMOSFET Transistors. This type is better than 1 & 2, because it reduces the consumption of power, and increases the processing speed; because of low resistive compared with NMOS & PMOS.

- Bipolar Junction Transistor (BJT).

Static CMOS Inverter

- XOR:

XOR gate is constructed using any type of MOSFETS Transistors.

- NOR:

NOR gate can be constructed using NMOS or CMOS FETS.

Unbuffered CMOS two inputs NOR gate.

- NAND:

NAND gate uses gates fewer than any other logical gate, so its cost is low. Also, NAND gate is Universal gate; since any gate can be implemented using NAND it is preferred to use NAND instead of other gate. We find that it is costs only £0.22-(£0.25). IC name: (NAND GATE, QUAD 2-INPUT, PDIP14 - SN74HC03N - TEXAS INSTRUMENTS).

CMOS NAND gate

TTL NAND gate

- Questions:

- Draw the logic diagram showing the implementation of the following Boolean equation using “AND” gates F = AB (CA).

Fig.(13)

- Draw the logic diagram of the following Boolean equations:

- F1= (A+B) (CD+A).

- F2= (ABC+D) C.

Fig.(14)

Fig.(15)

- Implement the OR operation using AND, NOT gate.

F=A+B = ((A+B)’)' = (A’.B’)’.

Fig.(16)

- Implement the AND gate using OR, NOT gate. Draw the logic diagram used in both cases and write Boolean equation.

F= A.B = ( (A.B)' )' = ( A'+B')' .

Fig.(17)

- Prove that the equality operation F l =AB+A’B’ is the inverse of exclusive OR operation F2=AB’+A’B (use De Morgan’s theorem).

F2'= (A’B+ AB’)'= (A'B)'. (AB')' = (A+B'). (A’+B)

=AA'+AB+A'B'+BB'

= AB + A'B' = F1.

- Suggest a logic diagram which will give a NAND gate with four inputs using two input NAND gates, Implement suggested network.

F = (ABCD)' = (AB)' + (CD)' = (( (AB)' + (CD)’)’)'= (((AB)')’. ((CD)')’)'

Fig.(18)

- Show how is it possible to reduce Boolean expressions by means of k_map.

Implement the minimal expressions using NAND gates.

- F1 = A'BCD + ABCD' + A'BCD' + ABCD'.

=m7+ m14+ m6+ m14 = ∑(6, 7, 14).

| 0 | 0 | 0 | 0 |
| --- | --- | --- | --- |
| 0 | 0 | 1 | 1 |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 0 |

K-map.

F1=A'BC+ BCD'.

(a)

Using NAND Gates:

(b)

Fig.(19)

- F2 = A'B'C'D' + AB'CD' + A'B'CD' + A'BC'D'.

=m0+ m10+ m2+ m4= ∑(0, 2, 4, 10).

| 1 | 0 | 0 | 1 |
| --- | --- | --- | --- |
| 1 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 |

K-map

F2= A'C'D'+ B'CD.

(a)

Using NAND Gates:

(b)

Fig.(20)

- REFRENCES:

- http://en.wikipedia.org/wiki/NAND_gate.

- http://en.wikipedia.org/wiki/AND_gate.

- http://en.wikipedia.org/wiki/OR_gate.

- http://en.wikipedia.org/wiki/NOT_gate.

- http://en.wikipedia.org/wiki/NOR_gate.

- http://en.wikipedia.org/wiki/XOR_gate.

- http://cpc.farnell.com/1/3/nand-gate.