Part 1:

Activity 1.1:

We firstly opened the cmd screen and we activated the debug command ,and we wrote the following instructions written in assembly language  at the offset memory locations 100h ,by typing A100 then  :

MOV AX, 2864

ADD AX, 3749

MOV BX, AX

SUB BX, 2805

NOP

Activity 1.2:

Then we used the command U to unassembled the instructions and we get the machine code that is represented in table (1) :

| Assembly code | Machine code |
| --- | --- |
| MOV AX, 2864 | B86428 |
| ADD AX, 3749 | 054937 |
| MOV BX, AX | 89C3 |
| SUB BX, 2805 | 81EB0528 |
| NOP | 90 |

Table (1)

We notice that we get the machine code for each instruction ,

Activity 1.3: by this command we also can get the number of bytes needed for each instruction ,which is shown in table (2)

| Assembly code | Machine code |
| --- | --- |
| MOV AX, 2864 | 3 |
| ADD AX, 3749 | 3 |
| MOV BX, AX | 2 |
| SUB BX, 2805 | 4 |
| NOP | 1 |

Table (2)

As we note the number of bytes is differ from one instruction to another.

Activity (1.4):

The immediate data value (2864) stored in location 101 h , by situate :

64 in location 101 h

28 in location 102 h

This result is dependent that the system is little Indian, so the lowest byte is put in the first location, and the highest one on the next one …

Activity 1.5:

Then we used the command R, which we used to determine the contents  of the registers as shown in table (3) below:

| Register | Content |
| --- | --- |
| CS | 1362 |
| IP | 0100 |
| AX | 0000 |
| BX | 0000 |

Table (3)

We note from the results we get in the previous table , since we didn’t execute the instructions yet , the value of code segment represent  the location of the code , the instruction pointer (IP) represent where we want  to be the code segment which is location 100h that we wrote our instructions from , the other registers  (AX,BX) have no initial values  yet…

Activity 1.6:   The predictions of the contents of CS, IP, AX and BX:

| Register | MOV AX, 2864 | ADD AX, 3749 | MOV BX, AX | SUB BX, 2805 |
| --- | --- | --- | --- | --- |
| CS | 1362 | 1362 | 1362 | 1362 |
| IP | 0103 | 0106 | 0108 | 010C |
| AX | 2864 | 5FAD | 5FAD | 5FAD |
| BX | 0000 | 0000 | 5FAD | 37A8 |

Table (4)

We note from the results above that the expected  value of the instruction pointer is changing or increasing by the same number of bytes for each instruction as shown in table (2) , and also the  expected values of AX , BX is changing according to the instructions will  be  executed .

Activity 1.7:

After executing the instructions by command T  ,the real contents of CS, IP, AX and BX are as follows:

| Register | MOV AX, 2864 | ADD AX, 3749 | MOV BX, AX | SUB BX, 2805 |
| --- | --- | --- | --- | --- |
| CS | 1362 | 1362 | 1362 | 1362 |
| IP | 0103 | 0106 | 0108 | 010C |
| AX | 2864 | 5FAD | 5FAD | 5FAD |
| BX | 0000 | 0000 | 5FAD | 37A8 |

Table (5)

We note that the real is identical to the predict values , to the reasons we mentioned in the last activity …