بسم الله الرحمن الرحيم

Computer Systems Engineering Department

ENCS 212

Digital Electronics And Computer Organization Lab

Report  For Experiment NO.12

Arithmetic Logic Unit (ALU) Circuit

- Student Name: Mohammad Wahbeh

- ID#: 1082008

- Student Name: Dania Odeh

- ID#: 1070250

- Instructor Name:  Dr. Mohammad Abu Ayash

- Section: 2

In this experiment we want to understand functions and applications of the ALU, or arithmetic logic unit. and perform arithmetic and logic operations using the 74181 ALU IC.

Then  understand the construction and applications of parity generators. And  generate parity generator IC.

- ALU :

In this experiment the 74181 ALU IC will be used to introduce the concept of

ALU. Its logic diagram is shown in Fig1 .

Fig. 1 :  ALU block diagram

It consists of two major parts: the arithmetic unit and the logic unit. The output,

either arithmetic or logic, is selected by a multiplexer (MUX). S2 is the selector

gate on the MUX and its state will determine the output of the ALU.

When S2=0, arithmetic operation is executed.

When S2=1, logic operation is executed.

Fig. 2 shows the pin assignment and Table 1 is the function table for the 74181.

Fig. 2 :  74181 pin assignment

The 74181 has two 4-bit inputs A and B, as well as a "carry-in" (CA) input.

The purpose of CA is to provide a reverse carry signal (CA=0 when there is a carry). There is a mode control input (M) and 4 function-select lines S0, S1, S2, S3, forming sixteen logic or arithmetic operations.

The 74181 also has a 4-bit output (F3-F0), a "carry-out" or "Cn+4" output; G (Generate) and P (Propagate) output. Refer to the truth table of the 74181 in Table 1.

The "+" symbol means OR logic, "Plus" means the sum of inputs. The major advantage of the 74181 is its ability to perform arithmetic functions such as addition; subtraction; shifting; and logic functions such as AND, OR and XOR functions.

The mode control input (M) and function-select lines (S0-S3) determines which function it will perform.

The mode control for 74181 is determined by these factors:

1. Addition: A "0" is generated at CA input to signify the existence of a carry.

After the arithmetic operation, if the sum is larger than 15, "0" will be

generated at CA again.

2. Subtraction: A "0" is generated at Cn+4 if the result is "0" or positive. If "0"

is generated at CA, the result is negative or there is a borrow. If the result of

subtraction is negative, for example "-4", the 4-bit output F will be in 2's

complement and Cn+4=1.

We set the KL-26005 Module on the KL-22001 Basic Electricity Circuit Lab, and located block b. Then , we connected function-select lines S3-S0 to Data Switches  SW7- SW4 respectively. Also , we connected M to SW3 for selecting arithmetic or logic operation.

When M="0" arithmetic operation is performed. When M="1" logic function is performed.

We connected inputs A3-A0 to D7-D4 and B3-B0 to D3-D0; connected Cn to "0"; outputs

F3-F0 to Logic Indicators L4-L1 and Cn+4 to L8.

Fig. 4 Wring diagram (KL-26005 block b, U4=74181)

We set M to "1" to perform the following logic functions:

(1) When S3S2S1S0= 0000, A3A2A1A0=0000 and B3B2B1B0=1111, the

outputs F3F2F1F0=   1111  =  (A +B).

(2) When S3S2S1S0= 0000, A3A2A1A0=1100 and B3B2B1B0=1010, the

outputs F3F2F1F0=   0011 = (A)’ .

(3) When S3S2S1S0=1001, A3A2A1A0=1100 and B3B2B1B0=0110, the

outputs F3F2F1 F0=  0101 =  ( B)’

(4) When S3S2S1S0=1011, A3A2A1A0=0011 and B3B2B1B0=1001, the

outputs F3F2F1 F0=   0001 = A B.

We set M to "0", Cn to "0" and ignored the previous carry. And performed the following arithmetic functions:

(1) When S3S2S1S0=1001, A3A2A1A0=B3B2B1B0=0100, the outputs

F3F2F1 F0 =  1001 = (A plus B plus 1) and Cn+4 = 1.

(2) When S3S2S1S0=1001, A3A2A1A0=1000 and B3B2B1B0=1100, the

outputs F3F2F1F0 = 0101 = ( A plus B plus 1 )  and Cn+4 =  0 .

(3) When S3S2S1S0=0011, A3A2A1A0=0100 and B3B2B1B0=0010, the

outputs F3F2F1 F0 =  0000 = (  zero )    and Cn+4 =0 .

(4) When S3S2S1S0=0011, A3A2A1A0=1010 and B3B2B1B0=1000, the

outputs F3F2F1 F0 =   0000   = (zero ) and Cn+4 = 0  .

(5) When S3S2S1S0=0000, A3A2A1A0=1010 and B3B2B1B0=0011 , the

outputs F3F2F1 F0 =  1011= ( A plus 1  )  and Cn+4 = 1 .

Note : we noticed that Cn + 4 is active low .

We set M to "0" and Cn to "1", followed the input sequences in table 2 and recorded the outputs. Depending on the state of M and Cn, function-select line S0-S3 has different functions .

| F0 | F1 | F2 | F3 | Cn+4 | A0 | A1 | A2 | A3 | B0 | B1 | B2 | B3 | S0 | S1 | S2 | S3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |

Note that we found some errors in this table that don’t match the function table of the ALU (table 1 of  the thoeritical experement) , fore example  :

1.When S3S2S1S0= 0000, A3A2A1A0=0001 and B3B2B1B0=0001, the

outputs  was F3F2F1F0=   1001 != 0010 !=  (A plus 1) .

2.When S3S2S1S0= 0000, A3A2A1A0=0010 and B3B2B1B0=0011, the

outputs  was F3F2F1F0=   0011 != 0100 !=  (A plus 1) .

3.When S3S2S1S0= 0111, A3A2A1A0=1000 and B3B2B1B0=0111, the

outputs  was F3F2F1F0=   1111 != 1000 !=  (A B’) .

- Parity Generator Circuit

A parity bit, generated by the parity generator, usually accompanies the data transmission process. The parity bit provides as a reference point and allows us to compare and check whether the transmission process and the data transmitted are correct or not.

There are two types of parity generators. The "Odd" parity generator will generate a "1" if the data contains an even number of "1"s. For example, data "10111011" has six "1 "s. When the parity bit is added to the end of this data, the number of "1 "s in the data will become an "Odd" number, hence the name "Odd Parity Generator", On the other hand, an Even parity generator will add a "1" to data with odd number of "1"s to make the total number of "1"s even. If the data already has an even number of "1"s no parity bit is generated. Output Y of the Even parity

generator shown  will be 0 if the inputs ABCDEFGH is equal to 10111011.

We connected inputs A, B, C, D, and E to Data Switches SW0-SW4 and output F6 to logic Indicator L1.And followed the input sequences in the following Table and recorded the outputs.

| INPUTS |  |  |  |  | OUTPUT |
| --- | --- | --- | --- | --- | --- |
| E | D | C | B | A | F6 |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 0 | 1 | 0 | 1 |
| 0 | 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

C. Parity Generator Ic :

We set the KL-26005 Module on the KL-22001 Basic Electricity Circuit Lab, and located block c. U7 is a parity generator IC.

We connected inputs A1, B1, C1, D1, E1. F1, G1. H1. 11 to Data Switches SW0- SW7 and D7 respectively. And connected outputs Y0 to L1 and Y1 to L2. Then , we applied +5VDC from the Fixed Power on KL-22001 Lab to KL-26002 Module.

Fig. 7 : Wiring diagram (KL-26005 block c)

We followed the input sequences given in Table 4and recorded the outputs.

| INPUTS |  |  |  |  |  |  |  |  | Y0 | Y1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L | H | G | F | E | D | C | B | A | EVEN | ODD |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 |

CONCLUSION

1. The 74181 has 16 arithmetic functions with or without carry, at the same time it is also capable of performing various logic functions. Due to time limitation we did not explore each and every function of the 74181. Such a complicated device is not easy to use unless it is

controlled by a computer or a microprocessor.

2. There are two types of parity generators: Odd and Even.