- Procedure:

- Constructing 4-to-2-Line Encoder with Basic Gates.

First, we set the KL-26003 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block a. Then, we complete the connections by referring to the wiring diagram in Figure.1, and apply +5 VDC from the Fixed Power on KL-22001 Lab to KL-26003 Module.  Connect inputs A, B, C & D to Data Switches SW0-SW3 respectively; outputs F8 and F9 to Logic Indicator L0 and L1.

Figure.1: wiring diagram of 4-to-2 line Encoder.

Then, we follow the input sequence D, C, B, A, and record the results in Table.(1).

| D | C | B | A | F9 | F8 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 |

Table (1)

We note that this circuit works as 4 to 2 line encoder only when one of the input=1, and in the other cases the output equals to zero.

Input (DCBA): output (F9F8).

0001: 00.

0010: 01.

0100: 10.

1000: 11.

- Constructing 4-to -2 priority encoder.

Then, we construct the circuit of 4- to -2 priority encoder as shown in fig.2(a), with only nand gates (2-inputs nand) as shown in fig.2 (b).

Then, set values to D3, D2, D1 and D0 as in Table. (2), and record the values of output y & x.

Figure.2 (a)

Figure.2 (b)

| D0 | D1 | D2 | D3 | X | Y |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

Table (2)

We note that this circuit works as priority encoder; since the priority is as

follow:

When D3 is 1 XY = 11.

When D2 is 1 & D3=0 XY = 10.

When D1 is 1 & D2=0 & D3=0 XY = 01.

When D0 is (1 or 0) & D1=0 & D2=0 & D3=0 XY = 00.

- Constructing 10-to-4-Line Encoder with TTL IC.

First, we set the KL-26004 Module on the KL-22001 Basic Electricity Circuit Lab,

and locate block a, as shown in fig. (3). Then, we apply +5 VDC from Fixed Power

to KL-26004. And connect inputs A1-A8 to SW0-SW7 and A9 to D7. Then, connect

outputs F1-F4 to Logic Indicators L1-L4. And follow the input sequences given in

Table.3 and record the output.

Figure.3: (74147) BCD Priority Encoder.

| A9 | A8 | A7 | A6 | A5 | A4 | A3 | A2 | A1 | F1 | F2 | F3 | F4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |

Table (4)

We note that this IC works as BCD priority encoder, with active low input and

output. With priority given as follow A9, A8, A7, A6, A5, A4, A3, A2 then A1;

since A9 given the highest priority  when A9= 0, F1F2F3F4=0110 = (1001)'.

- Constructing 2-to4-Line Decoder with Basic Gates.

First, we set the KL-26003 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block c. Then, Apply +5 VDC from the Fixed Power on KL-22001 Lab to KL-26003 Module. After that, we connect inputs A and B to Data Switches SW0 and SW1. And outputs 1-F4 to Logic Indicators L0-L3 respectively. Then, Follow the input sequences for A and B in Table.5 and record output.

Figure.4: 2-to-4 Decoder (KL_26003 block C).

| B | A | F1 | F2 | F3 | F4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 |

Table (5)

We note that this circuit works as 2 to 4 decoder (Inputs & outputs are booth Active high).

- Constructing 4-to-10-Line Decoder with TTL IC.

We set the KL-26002 module on the Kl-22001 Basic Electricity Circuit Lab, and locate block c. Then, we connect inputs A1, B1, C1, D1 to the Data Switches SW0, SW1, SW2, and SW3, respectively. And connect outputs 0-9 to Logic Indicator L0-L9.  After that, we apply +5 VDC from the Fixed Power on KL-

22001 Lab to KL-26002 Module. And follow the input sequences for A- D as in

Table.6 and record the output in each case.

Figure.5: 4-to-10 line Decoder.

| BCD | D | C | B | A | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 3 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 5 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 7 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |

Table (6)

This chip is BCD decoder and its input is active high, but the output is active

Low; since one of its outputs is activated (equals zero) at each input sequence

(According to the value of the input), i.e. when input (DCBA) = 1 (in BCD)

Output line #1= 0 (active), and other lines = 1 (inactive).