بسم الله الرحمن الرحيم

Computer System Engineering Department

ENCS 211

DIGITAL ELECTRONICS AND COMPUTER ORGANIZATION LAB

Report for Experiment NO.3

Encoders, Decoders, Multiplexers and De-multiplexers.

- Students name & ID: Hanadi Mzahim, 1071152 & Eman Arar, 1070720.

- Instructor Name:  MR. Mohammad Abu-Ayyash.

- Teacher Assistant: ENG. Anas Abd-elrazeq.

- Date: 10-2-2010.

- Section: 2.

- Procedure:

- Constructing 4-to-2-Line Encoder with Basic Gates.

First, we set the KL-26003 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block a. Then, we complete the connections by referring to the wiring diagram in Figure.1, and apply +5 VDC from the Fixed Power on KL-22001 Lab to KL-26003 Module.  Connect inputs A, B, C & D to Data Switches SW0-SW3 respectively; outputs F8 and F9 to Logic Indicator L0 and L1.

Figure.1: wiring diagram of 4-to-2 line Encoder.

Then, we follow the input sequence D, C, B, A, and record the results in Table.(1).

| D | C | B | A | F9 | F8 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 |

Table (1)

We note that this circuit works as 4 to 2 line encoder only when one of the input=1, and in the other cases the output equals to zero.

Input (DCBA): output (F9F8).

0001: 00.

0010: 01.

0100: 10.

1000: 11.

- Constructing 4-to -2 priority encoder.

Then, we construct the circuit of 4- to -2 priority encoder as shown in fig.2(a), with only nand gates (2-inputs nand) as shown in fig.2 (b).

Then, set values to D3, D2, D1 and D0 as in Table. (2), and record the values of output y & x.

Figure.2 (a)

Figure.2 (b)

| D0 | D1 | D2 | D3 | X | Y |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |

Table (2)

We note that this circuit works as priority encoder; since the priority is as

follow:

When D3 is 1 XY = 11.

When D2 is 1 & D3=0 XY = 10.

When D1 is 1 & D2=0 & D3=0 XY = 01.

When D0 is (1 or 0) & D1=0 & D2=0 & D3=0 XY = 00.

- Constructing 10-to-4-Line Encoder with TTL IC.

First, we set the KL-26004 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block a, as shown in fig. (3). Then, we apply +5 VDC from Fixed Power to KL-26004. And connect inputs A1-A8 to SW0-SW7 and A9 to D7. Then, connect outputs F1-F4 to Logic Indicators L1-L4. And follow the input sequences given in Table.3 and record the output.

Figure.3: (74147) BCD Priority Encoder.

| A9 | A8 | A7 | A6 | A5 | A4 | A3 | A2 | A1 | F1 | F2 | F3 | F4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |

Table (4)

We note that this IC works as BCD priority encoder, with active low input and output. With priority given as follow A9, A8, A7, A6, A5, A4, A3, A2 then A1; since A9 given the highest priority  when A9= 0, F1F2F3F4=0110 = (1001)'.

- Constructing 2-to4-Line Decoder with Basic Gates.

First, we set the KL-26003 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block c. Then, Apply +5 VDC from the Fixed Power on KL-22001 Lab to KL-26003 Module. After that, we connect inputs A and B to Data Switches SW0 and SW1. And outputs F1-F4 to Logic Indicators L0-L3 respectively. Then, Follow the input sequences for A and B in Table.5 and record output.

Figure.4: 2-to-4 Decoder (KL_26003 block C).

| B | A | F1 | F2 | F3 | F4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 |

Table (5)

We note that this circuit works as 2 to 4 decoder (Inputs & outputs are booth Active high).

- Constructing 4-to-10-Line Decoder with TTL IC.

We set the KL-26002 module on the Kl-22001 Basic Electricity Circuit Lab, and locate block c. Then, we connect inputs A1, B1, C1, D1 to the Data Switches SW0, SW1, SW2, and SW3, respectively. And connect outputs 0-9 to Logic Indicator L0-L9.  After that, we apply +5 VDC from the Fixed Power on KL-

22001 Lab to KL-26002 Module. And follow the input sequences for A- D

as in Table.(6) and record the output in each case.

Figure.5: 4-to-10 line Decoder.

| BCD | D | C | B | A | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 3 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 5 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 7 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |

Table (6)

This chip is BCD decoder and its input is active high, but the output is   active Low; since one of its outputs is activated (equals zero) at each input sequence (According to the value of the input), i.e. when input (DCBA) = 1 (in BCD)  Output line #1= 0 (active), and other lines = 1 (inactive).

- Constructing BCD-to-7-Segment Decoder.

We set the KL-26003 Module on the KL-22001 Basic Electricity Circuit Lab,    and locate block, and we Applied +5 VDC from the Fixed Power on KL-22001 Lab to KL-26003 Module. The U5 7448 is aBCD-to-7-segment decoder/driver with internal pull-up outputs.

Figure.6: Wiring diagram (KL-26003 block b)

Then we connected BCD inputs D, C, B, A to Data Switches SW3, SW2, SW1, SW0, respectively. And we connect decoder outputs a - g to DP1 inputs a – g respectively Connect "RBI" to SW7, "LT" toSW6 and "BI/RBO" to Logic Indicator L0. We Set RBI= "1" and LT="1". For the input sequences for D, C, B, A in Table.7 we recorded the outputs of the 7-segment display DP1.

When we set LT="0", we note that the 7 segment display number (8) all the time we change the input (a, b, c, d, e, f, g = 1).

When we set RBI = “0” and LT =1, we note that nothing displayed on DB1 despite a trash, as we change the input.

| D C B A | Display Pattern |
| --- | --- |
| 0 0 0 0 | 0 |
| 0 0 0 1 | 1 |
| 0 0 1 0 | 2 |
| 0 0 1 1 | 3 |
| 0 1 0 0 | 4 |
| 0 1 0 1 | 5 |
| 0 1 1 0 | 6 |
| 0 1 1 1 | 7 |
| 1 0 0 0 | 8 |
| 1 0 0 1 | 9 |
| 1 0 1 0 | - |
| 1 0 1 1 | - |
| 1 1 0 0 | - |
| 1 1 0 1 | - |
| 1 1 1 0 | - |
| 1 1 1 1 | - |

Table .7

We note from the previous table that U5 works as a decoder, for each certain input it enable parts of the output, we note that this decoder shows values until “9”, and it is an active high system.

- Constructing 2-to-1-Line Multiplexer with basic Gates

We set the KL-26004 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block e. We applied +5 VDC from the Fixed Power on KL-22001 Lab to KL- 26004 Module.

Figure.7: 2-to-1 Multiplexer

We Connected inputs A, B to Data Switches SW0, SW1; selector C to SW2 and the Output F3 to Logic Indicator L0. We followed the input sequences in Table.8 and we recorded the states of F3 which input (A or B) determines the output when C=O? _B____. Which input (A or B) determines the output when C=1? _A__.

| C | B | A | F |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 |

Table (8)

We note from the previous table that this multiplexer that every value for the selection line give us a one output, in each time it  shows one of the inputs . When c =0, output shows b, when c =1, output = A.

- Constructing 8-to-1-Line Multiplexer with TTL IC

We set the KL-26004 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block f, we applied +5VDC from the Fixed Power on KL-22001 Lab to KL-26004 Module.

Figure.8: 8-to-1 MUX (KL-26004 block f)

Refer to the data sheet for specifications of the 74151, U6

When CBA ="000", data at D0 is send to output Y.

When CBA ="010", data at D2 is send to output Y.

When CBA ="111", data at D7 is send to output Y.

The IC will function properly only when STROBE ="0".

Y will remain “0” when STOROBE=”1”.

We connected inputs D0~D7 to Data Switches D0~D7; input C, B, A to Data SwitchesSW2, SW1, SW0. And STROBE to Data Switch SW3. And output Y and F to Logic Indicators L0 and L1, respectively. Set SW3 to "0". We followed the input sequences in Table.9. Then we switched D0~D7 and recorded the output states.

| C | B | A | Y | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 |

Table.9

Since we choose the input sequence (10011101) from (D7 _D0 ),the output is recorded according  to that sequence , for example D0 =1 from the input , when the selection i =000 the output =1.

- Using Multiplexer to Create Function

We set the KL-26004 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block f, and we applied +5 VDC from the Fixed Power on KL-22001 Lab to KL-26004 Module.

Figure.9: Wiring diagram (KL-26004 block f)

The U6 (74151) will used to create the function:

F (D, C, B, A) =Σ(0,2,4,5,7,8,10,11,15) 3. Sketch on (Figure.9) the input connections necessary to implement the function F (D, C, B, A) =Σ(0,2,4,5,7,8,10,11,15)We  Completed  the connection by referring to the wiring diagram in Figure.9, and we connect inputs D, C, B, A to Data Switches SW3, SW2, SW1, SW0 respectively. Output Y was connected to Logic Indicator L0. We followed the input sequences inTable.10 and recorded the output states.

D C B A Y

0 0 0 0 1

0 0 0 1 0

0 0 1 0 1

0 0 1 1 0

0 1 0 0 1

0 1 0 1 1

0 1 1 0 0

0 1 1 1 1

1 0 0 0 1

1 0 0 1 0

1 0 1 0 1

1 0 1 1 1

1 1 0 0 0

1 1 0 1 0

1 1 1 0 0

1 1 1 1 1

Table.10

If we have a 8x1 mux ,we consider D,C,B  to be the selection lines and  A for the input  We can note from the previous table  that there is a relation between A and the output ,we have 8 combinations, A', A', 1, A, A', 1, 0, A.

- Constructing 1-to-2-Line Demultiplexer with Basic Logic Gates

We set the KL-26004 Module on the KL-22001 Basic Electricity Circuit Lab, and located block e. We completed the connection by referring to the wiring diagram in Figure.15.we Applied +5VDC from the Fixed Power on KL-22001 Lab to KL-26004 Module.

Figure.10: 1-to-2 Demultiplexer

We connected a to Data Switch SW0; C to SW3; F1 and F2 to Logic Indicators L0and L1 respectively. We set C to "0", change A and observe outputs F1 and F2.  when A=0, F1,F2 = 1,1  ,WHEN A=1,F1,F2 = 1,0. Then C to "1", change A and observe outputs F1 and F2. When A=1, F1,F2 = 1,1  ,WHEN A=1,F1,F2 = 0,1.

K. Constructing 1-to-8-Line Demultiplexer with CMOS IC

1. Set the KL-26004 Module on the KL-22001 Basic Electricity Circuit Lab, and locate block b as shown in Fig.11. Apply +5VDC from the Fixed Power on KL-22001 Lab to KL-26004 Module. U2 (4051) will be used in the experiment.

Figure.11: 1-to-8 Demultiplexer.

We Connected E and D to Data Switches D0 and D1, respectively We connected input A to SW0, B toSW1, C to SW2; outputs Y0~Y7 to Logic Indicators L0~L7, respectively. We  Set D=”0”, and we applied  the sequence 1-0-1-0 to the Common Input E and observe output Y0~Y7.Do the outputs change as the input sequence is?   ___yes______.

We set D="1", apply the input sequence 1-0-1-0 to the Common Input E and observe outputs Y0~Y7. Do the outputs change as the input sequence is applied? _no________. Which state of D changes the outputs? ______0____

We Set D="0". Using the same sequence for E (1-0-1-0), we followed the sequence for A, B and C given in Table.11. And we recorded the output states.

| A | C | B | Y0 y1 y2 y3 y4 y5 y6 y7 |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 0 0 0 0 0 0 0 |
| 1 | 0 | 0 | 0 1 0 0 0 0 0 0 |
| 0 | 0 | 1 | 0 0 1 0 0 0 0 0 |
| 1 | 0 | 1 | 0 0 0 1 0 0 0 0 |
| 0 | 1 | 0 | 0 0 0 0 1 0 0 0 |
| 1 | 1 | 0 | 0 0 0 0 0 1 0 0 |
| 0 | 1 | 1 | 0 0 0 0 0 0 1 0 |
| 1 | 1 | 1 | 0 0 0 0 0 0 0 0 |

Table.11

- Conclusion:

In this experiment, we implement the mux, decoder, encoder and demux using basic gates & using IC’s. Our work at beginning works well, but we found a problem when we use the KL-26004 Module; since it was fault and does not work correctly. Also, we use 4*1mux and 3*8decoder to implement different functions. Also we note that, by combining MUX and DMUX together, we can transmit signals for long distance in a system with increasing in the efficiency of transmission lines. Figure.12 shows a MUX-DMUX combinational circuit with 16 inputs, 16 outputs and 4 selectors.

Fig.12

- Problem:

Design a Majority Circuit; A circuit that takes 4 inputs and outputs 1 output, its output equals 1when 3 or 4 of the inputs are 1. You can only use two 4×1 multiplexers.

| A | B | C | D | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Table.12

At AB =00, F = 0.

At AB = 01, F= CD.

At AB = 10, F =CD.

At AB = 11, F =C+D.

Fig (12)