بسم الله الرحمن الرحيم

Computer System Engineering Department

ENCS 211

DIGITAL ELECTRONICS AND COMPUTER ORGANIZATION LAB

Report For Experiment NO.5

Sequential Logic Circuits.

- Students name & ID: Hanadi Mzahim, 1071152 & Eman Arar, 1070720.

- Instructor Name:  MR. Mohammad Abu-Ayyash.

- Teacher Assistant: ENG. Anas Abd-elrazeq.

- Date: 24-2-2010.

- Section: 2.

- Procedure:

5.5.1 Latches and Flip flops:

- Constructing RS latch with Basic Logic Gates.

We use KL-26006 module to implement the circuit of Fig.1.Then, we connect

switches to S and R inputs, and connect Q and Q’ output to logic indicator.

And follow the sequence of inputs (S & R), and fill Table.1.

Fig.1 (RS latch)

| S | R | Q | Q' | State |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | Indeterminate |
| 0 | 1 | 1 | 0 | Set Q |
| 1 | 0 | 0 | 1 | Reset Q |
| 1 | 1 | 0 | 1 | No change (Q) |

Table.1

We note that, this circuit is work as active low (input) SR latch; since there is no clock for synchronization, and the output has four states:

QQ'=11 (indeterminate state) when SR=00, and QQ'=10 (Set the output) when SR=01, and QQ'=01(Reset the output) when SR=10, and no change State QQ'=01 when SR=11; since in this case the latch maintains its previous value (Reset).

- Constructing RS latch with control input.

Then, we use KL-26006 module to connect the circuit shown in fig.2, by connecting CK2 (control input) to logic 1 (+5V), and S & R to switches s1, s0. And follow the input sequence in Table.2.

Fig.2 (RS Latch with control input)

| S | R | Q | Q' | State |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | No change (Q) |
| 0 | 1 | 0 | 1 | Reset Q |
| 1 | 0 | 1 | 0 | Set Q |
| 1 | 1 | 1 | 1 | Indeterminate |

Table.2

The circuit of fig.2 as we note from Table.2 works as in part A, but with active high SR (inputs).

- Constructing D latch with RS latch.

Then, we use KL-26006 module to connect the circuit shown in fig.3, by connecting CK2 (control input) to Pulse switch, and D to switch s0. And follow the input sequence in Table.3.

Fig.3

| CK2 | D | Q | Q' | State |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | No Change |
| 0 | 1 | 0 | 1 |  |
|  | 0 | 0 | 1 | Q= D Q'= D' |
|  | 1 | 1 | 0 |  |

Table.3

As we can see from Table.3 the value of Q =D only when we provide a pulse for the CK2; otherwise Q maintain its previous value.

- Constructing JK latch with RS latch

After that, we use KL-26006 module to connect the circuit of fig.4, by connecting CK2 to Pulse switch and J & K to switches s1 & s0, and Q, Q' to logic indicators. And follow the input sequence of Table.4.

Fig.4 (JK FF)

| CK2 | J | K | Q | Q' | State |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 1 | No change (Q) |
|  | 0 | 1 | 0 | 1 | Reset Q |
|  | 1 | 0 | 1 | 0 | Set Q |
|  | 1 | 1 | 0 | 1 | Complement(Q) |

Table.4

This type of latch (JK) works as SR latch in state 0(No change), 1(Reset) & 2(Set). But at state JK=11 it complements the previous value of Q & so Q'. As shown in Table.4.

- Constructing JK Flip-flop with master- slave RS latches.

After that, we use KL-26006 module to connect the circuit of fig.5, by connecting CK1 to Pulse switch and J & K to switches s1 & s0, and S, R, Q, Q' to logic indicators. And follow the input sequence of Table.5.

Fig.5

| CK1 | J | K | S | R | Q | Q' | -> | S | R | Q | Q' | State |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 1 | 0 | 1 | 0 | -> | 0 | 1 | 0 | 1 | No change |
|  | 0 | 1 | 0 | 1 | 0 | 1 | -> | 0 | 1 | 0 | 1 | Reset |
|  | 1 | 0 | 1 | 0 | 1 | 0 | -> | 1 | 0 | 1 | 0 | Set |
|  | 1 | 1 | 0 | 1 | 0 | 1 | -> | 0 | 1 | 0 | 1 | complement |
|  | 1 | 1 | 0 | 1 | 0 | 1 | -> | 0 | 1 | 0 | 1 | complement |

Table.5

The circuit of fig.5 is working as JK flip flop (works as JK latch, but it is synchronized), which constructed using master_ slave SR latch.

The data we have is correct compared with the theoretical ones in Parts A, B, C, D. But in part E we have a problem in complement state (JK=11 the second one); since at the first time it works correctly and the output is complemented, but at the next clock while JK still equals 11 the value of Q does not change. We told the teacher and he check the connections, but they are true. And we do not know why this happens; although we try many times to found the error. The incorrect values are shadowed in Table.5.

5.5.2 Registers

- Constructing Shift Register with D Flip-Flops

Use KL-26006 module (block a) to construct the circuit shown in fig.6.

Connect A (serial input) to switch. Connect B (clear) to other switch. Connect

the outputs F1, F2, F3 and F4 to indication lamps. Connect CK to Pulser

Switch. Set B switch to 1 to clear the register, and then keep it on 0.  Set A to

1, then, apply three clock pulses to CK using the Pulser.

Fig.6. Shift Right Register

After doing the last step, we note that the value on the output

(F1F2F3F4=1110); since three clocks is applied to the shift right register.

After CLK1 F1F2F3F4=1000, After CLK2  F1F2F3F4=1100,

After CLK3 F1F2F3F4=1110.

To load 1011 value into the register we follow the sequence: 1, 1, 0 then 1.

F1F2F3F4=1011.

- 4-Bit Shift Register with serial and parallel load.

First, we connect the serial input (pin1) to one of the input switches. And connect the outputs QA, QB, QC and QD to indication lambs. Connect CLK1 (pin9) to the Pulser switch. Apply clock pulses to CLK1 and observe the serial data transfer. We load the register with 1101 value by following the input 1, 0, 1 then 1.

Then, we connect the inputs A, B, C, and D to switches. Connect the outputs QA, QB, QC and QD to indication lambs, and CLK1 (pin8) to the Pulser switch. Apply clock pulses to CLK1 and observe the parallel data transfer. Load the register with 1101 value. We do that by load the inputs ABCD with 1101 (Parallel mode).

Fig.7.IC7495 shift register with serial and parallel load.

    - Counters.

- 2-bit Synchronous Counter.

We use KL-26007 module, block c, which contains three separated JK flip flops to implement the 2-bit synchronous counter shown in fig.8. Connect CLK input to Pulser switch. Connect counter outputs Q1 and Q0 to indication lamps. Apply clock pulses to CLK input. Observe and record the outputs in Table.6 (a). Apply counter outputs Q1 and Q0 to D1 display inputs. Observe and record the outputs in Table.6 (b).

Figure.8: 2-bit Synchronous Counter

| CLK | Q1 | Q0 |
| --- | --- | --- |
|  | 0 | 0 |
|  | 0 | 1 |
|  | 1 | 0 |
|  | 1 | 1 |
|  | 0 | 0 |
|  | 0 | 1 |
|  | 1 | 0 |
|  | 1 | 1 |
| CLK | D1 |
|  | 0 |
|  | 1 |
|  | 2 |
|  | 3 |
|  | 0 |
|  | 1 |
|  | 2 |
|  | 3 |

- (b)

Tabble.6

We note that this counter counts from 0 to 3 then reset to zero then start counting again, also the counter is synchronous; since the same clock is connected to all flip      flops. The next value is set to the counter only when a clock is applied.

- 3-bit (divide-by-eight) Ripple Counter.

Use KL-26007 module, block c, which contains three separated JK flip flops to implement the 3-bit (divide by eight) Ripple counter shown in Figure.9. Connect CLK input to Pulser switch. Connect counter outputs Q2, Q1 and Q0 to indication lamps. Apply clock pulses to CLK input. Observe and record the outputs in Table.7 (a). Apply counter outputs Q2, Q1 and Q0 to D1 display inputs. Observe and record the outputs in Table.7 (b).

Fig.9: 3-bit Ripple Counter

| CLK | Q2 | Q1 |  | Q0 |
| --- | --- | --- | --- | --- |
|  | 0 | 0 |  | 0 |
|  | 0 | 0 |  | 1 |
|  | 0 | 1 |  | 0 |
|  | 0 | 1 |  | 1 |
|  | 1 | 0 |  | 0 |
|  | 1 | 0 |  | 1 |
|  | 1 | 1 |  | 0 |
|  | 1 | 1 |  | 1 |
|  | 0 | 0 |  | 0 |
|  | 0 | 0 |  | 1 |
| CLK | D1 |  |
|  | 0 |  |
|  | 1 |  |
|  | 2 |  |
|  | 3 |  |
|  | 4 |  |
|  | 5 |  |
|  | 6 |  |
|  | 7 |  |
|  | 0 |  |
|  | 1 |  |

- (b)

Tabble.7

We note that this counter counts from 0 to 7 then reset to zero then start counting again, also the counter is asynchronous; since not the same clock is connected to all flip flops. The next value is set to Q0 of the counter only when a clock is applied and other digits vary depending on the varying of previous digit.

- BCD Counter.

First, we locate BCD counter (IC 7490) on KL-26007 module, block b. Then, connect the circuit shown in Figure.10. Then, connect RO (1) and RO (2) (pins 2& 3) to 0v, note that RO (1) and RO (2) are two inputs to internal NAND gate which used to reset the counter. Connect clock INPUT 1 (pin 14) to Pulser switch. Connect the outputs A, B, C, and D to indication lamps. Apply clock pulses to INPUT 1, and observe the count sequence (0000-1001). Apply counter outputs A, B, C, and D to D1 display inputs. Apply clock pulses to INPUT 1. Observe the count sequence on the display.  Connect the INPUT 1 to the output of the clock generator. Observe the count sequence on the display.

Fig.10

The counter works perfectly and counts from 0000 to 1001 when a clock is

provided.

- Divide-by-8 counter using BCD chip counter.

Connect RO (2) (pin3) to +5V, and connect RO (1) (pin2) to QD (pin11) output. This will make counter reset after 111 (or 7) before the value of D becomes 1 at the first time (DCBA=1000//8) the pin reset of the counter is 0 (active low), so the counter is reset DCBA=0000, then starts the count again. Then, we connect clock INPUT 1 (pin 14) to Pulser switch. Connect the outputs A, B, C, and D to indication lamps. Apply clock pulses to INPUT 1, and observe the count sequence (0000-0111). Apply counter outputs A, B, C, and D to D1 display inputs. Apply clock pulses to INPUT 1.

- Tasks:

- Task1:

- We make a shift left register using IC7495 by taking the output in reverse order of the shift left mode; since the mode is shift right with output instead of F1F2F3F4 (to logic indicators L3L2L1L0) take them F4F3F2F1 to logic indicators L3L2L1L0.

- We make shift left operation using parallel mode by connecting F4F3F2F1 to logic indicators L3L2L1L0, and connecting the inputs ABCD  to F2F3F4 & SI(serial input) and load any value using serial input pin.

- Task2:

In order to make a three bit synchronous counter, we make the design below.

Design.1

We make this design in the lab, and it worked correctly.

- Task3:

- To make the BCD counter counts from 0000 to 0101, we reset the counter just before it becomes 6. We done that by connecting C & B pins to RO (1) & RO (2).

- To make the BCD counter counts from 0000 to 0100, we reset the counter just before it becomes 5. We done that by connecting C & A pins to RO (1) & RO (2).

- Discussion:

- Although latches are useful for storing binary information, they are rarely used in the sequential circuits design, why?

The latches are useful for storing information; since they are faster than flip flops (flip flops are master-slave latches), and there size also less than flip flop’s size and they consume less power. But they make many problems; they make the system asynchronous so increase the probability of glitches in the circuits that use them; since they are level triggering chips.

- What is the disadvantage of the RS flip flops?

The main disadvantages of SR latches is when S=R=1 in this state the latch make a set & reset to itself at the same time, so Q=Q'=1, and this is indeterminate state. So it is the responsibility of the designers to be ensure that S & R are not 1 at the same time

- What is the difference between “synchronous” and “ripple” counters?

- Ripple counter: the clock is not connected to all flip-flopsthere are no synchronization; since the output of flip-flop transition serves when the output of triggering flip-flop changes.

- Synchronous counter: there are common clock that connected to all flip-flops. All flip-flop’s output are adjusted only at the clock.

- References:

- http://www.answerbag.com/q_view/436819

- http://www.edaboard.com/ftopic109076.html

- http://www.cse.nd.edu/courses/cse221/www/labs/MG4_HD4.pdf

- Digital Design (Morris Mano).